SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

SN32F400 Series

USER’S MANUAL

SN32F407

SN32F405

SN32F403

SONiX 32-Bit Cortex-M0 Micro-Controller

SONIX reserves the right to make change without further notice to any products herein to improve reliability, function or design. SONIX does not  assume any liability arising out of the application or use of any product or circuit described herein; neither does it convey any license under its patent  rights nor the rights of others. SONIX products are not designed, intended, or authorized for us as components in systems intended, for surgical implant  into the body, or other applications intended to support or sustain life, or for any other application in which the failure of the SONIX product could create  a situation where personal injury or death may occur. Should Buyer purchase or use SONIX products for any such unintended or unauthorized  application. Buyer shall indemnify and hold SONIX and its officers, employees, subsidiaries, affiliates and distributors harmless against all claims, cost,  damages, and expenses, and reasonable attorney fees arising out of, directly or indirectly, any claim of personal injury or death associated with such  unintended or unauthorized use even if such claim alleges that SONIX was negligent regarding the design or manufacture of the part.

SONiX TECHNOLOGY CO., LTD Page 1 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

AMENDENT HISTORY

| Version | Date | Description |
| --- | --- | --- |
| 1.0 | 2024/04/10 | First version released. |
| 1.1 | 2024/05/15 | 1. Modify typing error of LVD Voltage Detector in 23.2 ELECTRICAL CHARACTERISTIC. 2. Add 15.8.3 I2C TX_BIT Changing Timing. 3. Add ELS operating note in 3.2.3 External Low-speed (ELS) Clock. |

SONiX TECHNOLOGY CO., LTD Page 2 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

Table of Content

AMENDENT HISTORY................................................................................................................................ 2 PRODUCT OVERVIEW....................................................................................................................... 16

1.1 FEATURES...................................................................................................................................... 16 1.2 SYSTEM BLOCK DIAGRAM ........................................................................................................ 18 1.3 CLOCK GENERATION BLOCK DIAGRAM................................................................................ 19 1.4 PIN ASSIGNMENT ......................................................................................................................... 20 1.5 PIN ALLOCATION TABLE............................................................................................................ 23 1.6 PIN DESCRIPTIONS....................................................................................................................... 25 1.7 PIN CIRCUIT DIAGRAMS............................................................................................................. 29 1.8 PIN CHARACTERISTICS............................................................................................................... 32

CENTRAL PROCESSOR UNIT (CPU) .............................................................................................. 33

2.1 MEMORY MAP............................................................................................................................... 33 2.2 SYSTEM TICK TIMER................................................................................................................... 34 2.2.1 OPERATION ............................................................................................................................ 34 2.2.2 SYSTICK USAGE HINTS AND TIPS ....................................................................................... 35 2.2.3 SYSTICK REGISTERS.............................................................................................................. 35 2.2.3.1 System Tick Timer Control and Status register (SYSTICK_CTRL)................................... 35 2.2.3.2 System Tick Timer Reload value register (SYSTICK_LOAD)........................................... 35 2.2.3.3 System Tick Timer Current Value register (SYSTICK_VAL)............................................ 36 2.2.3.4 System Tick Timer Calibration Value register (SYSTICK_CALIB) .................................. 36 2.3 NESTED VECTORED INTERRUPT CONTROLLER (NVIC) ..................................................... 37 2.3.1 INTERRUPT AND EXCEPTION VECTORS ........................................................................... 37 2.3.2 NVIC REGISTERS.................................................................................................................... 38 2.3.2.1 IRQ0~31 Interrupt Set-Enable Register (NVIC_ISER) ....................................................... 38 2.3.2.2 IRQ0~31 Interrupt Clear-Enable Register (NVIC_ICER) ................................................... 38 2.3.2.3 IRQ0~31 Interrupt Set-Pending Register (NVIC_ISPR) ..................................................... 38 2.3.2.4 IRQ0~31 Interrupt Clear-Pending Register (NVIC_ICPR) ................................................. 39 2.3.2.5 IRQ0~31 Interrupt Priority Register (NVIC_IPRn) (n=0~7)............................................... 39 2.4 APPLICATION INTERRUPT AND RESET CONTROL (AIRC).................................................. 39 2.5 CODE OPTION TABLE .................................................................................................................. 41 2.6 UNIQUE NUMBER......................................................................................................................... 41 2.7 CORE REGISTER OVERVIEW ..................................................................................................... 42

SYSTEM CONTROL............................................................................................................................. 43

3.1 RESET.............................................................................................................................................. 43 3.1.1 POWER-ON RESET (POR)...................................................................................................... 43

SONiX TECHNOLOGY CO., LTD Page 3 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

3.1.2 WATCHDOG RESET (WDT RESET)....................................................................................... 44 3.1.3 BROWN-OUT RESET............................................................................................................... 44 3.1.3.1 BROWN OUT DESCRIPTION ........................................................................................... 44 3.1.3.2 THE SYSTEM OPERATING VOLTAGE DECSRIPTION ............................................... 45 3.1.3.3 BROWN-OUT RESET IMPROVEMENT .......................................................................... 45 3.1.4 EXTERNAL RESET .................................................................................................................. 46 3.1.4.1 SIMPLY RC RESET CIRCUIT........................................................................................... 47 3.1.4.2 DIODE & RC RESET CIRCUIT ......................................................................................... 47 3.1.4.3 ZENER DIODE RESET CIRCUIT...................................................................................... 48 3.1.4.4 VOLTAGE BIAS RESET CIRCUIT................................................................................... 48 3.1.4.5 EXTERNAL RESET IC....................................................................................................... 49 3.1.5 SOFTWARE RESET ................................................................................................................. 49 3.2 SYSTEM CLOCK ............................................................................................................................ 50 3.2.1 INTERNAL RC CLOCK SOURCE ........................................................................................... 50 3.2.1.1 Internal High-speed RC Oscillator (IHRC).......................................................................... 50 3.2.1.2 Internal Low-speed RC Oscillator (ILRC)........................................................................... 50 3.2.2 EXTERNAL CLOCK SOURCE ................................................................................................ 50 3.2.2.1 External High-speed (EHS) Clock ....................................................................................... 50 3.2.3 External Low-speed (ELS) Clock ............................................................................................. 51 3.2.3.1 Bypass Mode ........................................................................................................................ 52 3.2.4 PLL ........................................................................................................................................... 53 3.2.4.1 PLL Frequency selection ...................................................................................................... 53 3.2.4.2 PLL Recommended Frequency settings............................................................................... 54 3.2.5 SYSTEM CLOCK (SYSCLK) SELECTION............................................................................... 54 3.2.6 CLOCK-OUT CAPABITITY..................................................................................................... 54 3.3 SYSTEM CONTROL REGISTERS 0.............................................................................................. 55 3.3.1 Analog Block Control register (SYS0_ANBCTRL)................................................................... 55 3.3.2 PLL control register (SYS0_PLLCTRL)................................................................................... 55 3.3.2.1 RECOMMEND FREQUENCY SETTING.......................................................................... 56 3.3.3 Clock Source Status register (SYS0_CSST).............................................................................. 56 3.3.4 System Clock Configuration register (SYS0_CLKCFG) .......................................................... 57 3.3.5 AHB Clock Prescale register (SYS0_AHBCP)......................................................................... 57 3.3.6 System Reset Status register (SYS0_RSTST) ............................................................................ 57 3.3.7 LVD Control register (SYS0_LVDCTRL)................................................................................. 58 3.3.8 External RESET Pin Control register (SYS0_EXRSTCTRL) ................................................... 58 3.3.9 SWD Pin Control register (SYS0_SWDCTRL)......................................................................... 59 3.3.10 Interrupt Vector Table Mapping register (SYS0_IVTM).......................................................... 59 3.3.11 Anti-EFT Ability Control register (SYS0_ANTIEFT)............................................................... 59 3.3.12 IHRC Frequency Adjustment register (SYS0_IHRCADJ)........................................................ 60 3.3.13 CT16Bn Clock Source Select register (SYS0_CT_CLKSEL) ................................................... 60

SONiX TECHNOLOGY CO., LTD Page 4 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

3.4 SYSTEM CONTROL REGISTERS 1.............................................................................................. 61 3.4.1 AHB Clock Enable register (SYS1_AHBCLKEN).................................................................... 61 3.4.2 APB Clock Prescale register 1 (SYS1_APBCP1)..................................................................... 62 3.4.3 Peripheral Reset register (SYS1_PRST)................................................................................... 63 3.4.4 ILRC Frequency Calibration register (SYS1_ILRCFC)........................................................... 64

SYSTEM OPERATION MODE........................................................................................................... 65

4.1 OVERVIEW..................................................................................................................................... 65 4.2 NORMAL MODE ............................................................................................................................ 65 4.3 LOW-POWER MODES................................................................................................................... 65

4.3.1 SLEEP MODE.......................................................................................................................... 65 4.3.2 DEEP-SLEEP MODE............................................................................................................... 66 4.4 WAKEUP ......................................................................................................................................... 67 4.4.1 OVERVIEW .............................................................................................................................. 67 4.4.2 WAKEUP TIME........................................................................................................................ 67 4.5 STATE MACHINE OF PMU........................................................................................................... 68 4.6 OPERATION MODE COMPARSION TABLE .............................................................................. 69 4.7 PMU REGISTERS ........................................................................................................................... 70 4.7.1 Power Control register (PMU_CTRL)..................................................................................... 70

GENERAL PURPOSE I/O PORT (GPIO).......................................................................................... 71

5.1 OVERVIEW..................................................................................................................................... 71 5.2 GPIO MODE .................................................................................................................................... 71 5.3 GPIO REGISTERS........................................................................................................................... 72

5.3.1 GPIO Port n Data register (GPIOn_DATA) (n=0,1,2,3)......................................................... 72 5.3.2 GPIO Port n Mode register (GPIOn_MODE) (n=0,1,2,3)...................................................... 72 5.3.3 GPIO Port n Configuration register (GPIOn_CFG) (n=0,1,2,3)............................................ 72 5.3.4 GPIO Port n Interrupt Sense register (GPIOn_IS) (n=0,1,2,3)............................................... 73 5.3.5 GPIO Port n Interrupt Both-edge Sense register (GPIOn_IBS) (n=0,1,2,3)........................... 74 5.3.6 GPIO Port n Interrupt Event register (GPIOn_IEV) (n=0,1,2,3)............................................ 74 5.3.7 GPIO Port n Interrupt Enable register (GPIOn_IE) (n=0,1,2,3)............................................ 74 5.3.8 GPIO Port n Raw Interrupt Status register (GPIOn_RIS) (n=0,1,2,3) ................................... 74 5.3.9 GPIO Port n Interrupt Clear register (GPIOn_IC) (n=0,1,2,3).............................................. 75 5.3.10 GPIO Port n Bits Set Operation register (GPIOn_BSET) (n=0,1,2,3).................................... 75 5.3.11 GPIO Port n Bits Clear Operation register (GPIOn_BCLR) (n=0,1,2,3)............................... 75

PERIPHERAL FUNCTION PIN ASSIGNMENT (PFPA)................................................................ 76

6.1 OVERVIEW..................................................................................................................................... 76 6.2 FEATURES...................................................................................................................................... 76 6.3 PIN ASSIGNMENT LIST................................................................................................................ 76 6.4 PFPA REGISTERS........................................................................................................................... 77

SONiX TECHNOLOGY CO., LTD Page 5 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

6.4.1 PFPA for CT16B0 register (PFPA_CT16B0) .......................................................................... 77 6.4.2 PFPA for CT16B1 register (PFPA_CT16B1) .......................................................................... 78 6.4.3 PFPA for CT16B5 register (PFPA_CT16B5) .......................................................................... 78 6.4.4 PFPA for UART0 register (PFPA_UART0)............................................................................. 79 6.4.5 PFPA for UART1 register (PFPA_UART1)............................................................................. 79 6.4.6 PFPA for I2C0 register (PFPA_I2C0)..................................................................................... 79 6.4.7 PFPA for SPI0 register (PFPA_SPI0) ..................................................................................... 80 6.4.8 PFPA for CMP register (PFPA_CMP).................................................................................... 80

DMA......................................................................................................................................................... 81

7.3.1 DMA Transactions.................................................................................................................... 82 7.3.2 Arbiter....................................................................................................................................... 82 7.3.3 Peripheral Mode....................................................................................................................... 83 7.3.4 DMA Channels ......................................................................................................................... 84

7.3.4.1 Programmable Data Sizes..................................................................................................... 84 7.3.4.2 Programmable Burst Sizes.................................................................................................... 84 7.3.4.3 Pointer Incrementation/Decrementation............................................................................... 84 7.3.4.4 Incremental Cyclic Mode ..................................................................................................... 84 7.3.4.5 Memory-to-Memory (M2M) Mode...................................................................................... 85 7.3.4.6 Memory-to-Peripheral (M2P) Mode .................................................................................... 85 7.3.4.7 Peripheral-to-Memory (P2M) Mode .................................................................................... 85 7.3.4.8 Peripheral-to-Peripheral (P2P) Mode ................................................................................... 85 7.3.4.9 Channel Configuration Precedure ........................................................................................ 85

7.3.5 Data Transfer Mapping Rule.................................................................................................... 86 7.3.5.1 Burst Data is Enough for Packet Operation.......................................................................... 86 7.3.5.2 Burst Data is Not Enough for Packet Operation................................................................... 87 7.3.5.3 Pack Function is Not Allowed to Decrement Type.............................................................. 88 7.5.12.1 SRC_RS/DST_RS Table .................................................................................................. 99

16+6 CHANNEL ANALOG TO DIGITAL CONVERTOR (ADC)................................................ 101

8.1 OVERVIEW................................................................................................................................... 101 8.2 FEATURES.................................................................................................................................... 102 8.3 ADC CONVERTING TIME .......................................................................................................... 102 8.4 ADC OFFSET CALIBRATION..................................................................................................... 103 8.5 ADC CONTROL NOTICE............................................................................................................. 103

8.5.1 ADC SIGNAL.......................................................................................................................... 103 8.5.2 ADC OVERRUN..................................................................................................................... 103 8.5.3 ADC PROGRAM .................................................................................................................... 103 8.5.4 ADC PIN CONFIGURATION................................................................................................ 104 8.6 ADC CONVERSION MODES ...................................................................................................... 104

SONiX TECHNOLOGY CO., LTD Page 6 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

8.6.1 Single Mode ............................................................................................................................ 105 8.6.2 Continuous Mode.................................................................................................................... 105 8.7 ADC WATCHDOG WINDOW ..................................................................................................... 106 8.8 ADC CIRCUIT............................................................................................................................... 107 8.9 ADC CONVERSION TRIGGER SOURCE ............................................................................................. 107 8.10 DMA MODE .................................................................................................................................... 107 8.10.1 Flow Chart.............................................................................................................................. 108 8.10.2 DMA Configuration Recommendations.................................................................................. 108 8.11 ADC REGISTERS.......................................................................................................................... 109 8.11.1 ADC Management register (ADC_ADM)............................................................................... 109 8.11.2 ADC Data register (ADC_ADB) ............................................................................................ 110 8.11.3 ADC Interrupt Enable register (ADC_IE).............................................................................. 110 8.11.4 ADC Raw Interrupt Status register (ADC_RIS)..................................................................... 111 8.11.5 ADC Raw Interrupt Status register (ADC_IC)....................................................................... 111 8.11.6 ADC Convert Control register (ADC_CONVCTRL).............................................................. 112 8.11.7 ADC Window Watchdog register (ADC_AWW)..................................................................... 112 8.11.8 ADC Window Watchdog Threshold register (ADC_AWWTH) .............................................. 113 8.11.9 ADC Management register 1 (ADC_ADM1).......................................................................... 113 8.11.10 ADC Calibration Status register (ADC_CALIST).............................................................. 113 8.11.11 ADC DMA Mode register (ADC_DMA)............................................................................. 114

RAIL TO RAIL ANALOG COMPARATOR ................................................................................... 115

9.1 OVERVIEW................................................................................................................................... 115 9.2 NORMAL COMPARATOR MODE.............................................................................................. 117 9.2.1 COMPARATOR ENABLE ...................................................................................................... 117 9.2.2 CMnOUT, CMnG AND CMnIF.............................................................................................. 117 9.2.3 COMPARATOR OUTPUT DEBOUNCE TIME CONTROL.................................................. 118 9.3 COMPARATOR APPLICATION NOTICE.................................................................................. 119 9.4 COMPARATOR REGISTERS .......................................................................................................... 120 9.4.1 CMP Control register (CMP_CTRL) ..................................................................................... 120 9.4.2 CMP Control register 1 (CMP_CTRL1)................................................................................ 121 9.4.3 CMP Internal Reference Voltage Source register (CMP_VIREF)......................................... 121 9.4.4 CMP Output Status register (CMP_OS) ................................................................................ 122 9.4.5 CMP Interrupt Enable register (CMP_IE)............................................................................. 122 9.4.6 CMP Raw Interrupt Status register (CMP_RIS).................................................................... 123 9.4.7 CMP Interrupt Clear register (CMP_IC)............................................................................... 123 9.4.8 CMP Output Debounce register (CMP_DB).......................................................................... 123

OPERATIONAL-AMPLIFIER (OPA).......................................................................................... 125 10.1 OVERVIEW................................................................................................................................... 125

SONiX TECHNOLOGY CO., LTD Page 7 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

10.2 CONFIGURATION OF OPERATION.......................................................................................... 126 10.3 OPA REGISTERS .......................................................................................................................... 127 10.3.1 OPA Control register (OPA_CTRL)....................................................................................... 127 10.3.2 OPA PGA Control register (OPA_PGACTRL)...................................................................... 127 10.3.3 OPA PGA Control register 2 (OPA_PGACTRL2)................................................................. 128

16-BIT TIMER WITH CAPTURE FUNCTION .......................................................................... 129

11.1 OVERVIEW................................................................................................................................... 129 11.2 FEATURES.................................................................................................................................... 129 11.3 PIN DESCRIPTION....................................................................................................................... 129 11.4 BLOCK DIAGRAM....................................................................................................................... 130 11.5 TIMER OPERATION .................................................................................................................... 131

11.5.1 Edge-aligned Up-counting Mode ........................................................................................... 131 11.5.2 Edge-aligned Down-counting Mode....................................................................................... 132 11.5.3 Center-aligned Counting Mode .............................................................................................. 132

11.6 PWM............................................................................................................................................... 133 11.6.1 PWM Mode 1 .......................................................................................................................... 133 11.6.2 PWM Mode 2 .......................................................................................................................... 134

11.7 INVERSE PWM OUTPUT WITH DEAD-BAND PERIOD ......................................................... 135 11.8 DMA MODE .................................................................................................................................. 137 11.8.1 Flow Chart.............................................................................................................................. 137 11.8.2 DMA Configuration Recommendations.................................................................................. 138 11.9 CT16B0 BREAK FUNCTION....................................................................................................... 138 11.9.1 Break by Comparator n Output.............................................................................................. 139 11.9.2 Break by Break Pin................................................................................................................. 139 11.10 CT16BN REGISTERS................................................................................................................ 141 11.10.1 CT16Bn Timer Control register (CT16Bn_TMRCTRL) (n=0,1,5) .................................... 141 11.10.2 CT16Bn Timer Counter register (CT16Bn_TC) (n=0,1,5) ................................................ 141 11.10.3 CT16Bn Prescale register (CT16Bn_PRE) (n=0,1,5)........................................................ 141 11.10.4 CT16Bn Prescale Counter register (CT16Bn_PC) (n=0,1,5)............................................ 142 11.10.5 CT16Bn Count Control register (CT16Bn_CNTCTRL) (n=0,1,5)..................................... 142 11.10.6 CT16Bn Match Control register (CT16Bn_MCTRL) (n=0,1,5) ........................................ 142 11.10.7 CT16Bn Match register 0~3 (CT16Bn_MR0~3) (n=0,1,5)................................................ 143 11.10.8 CT16Bn Capture Control register (CT16Bn_CAPCTRL) (n=0,1,5) ................................. 144 11.10.9 CT16Bn Capture 0 register (CT16Bn_CAP0) (n=0,1,5) ................................................... 144 11.10.10 CT16Bn External Match register (CT16Bn_EM) (n=0,1,5) .............................................. 144 11.10.11 CT16Bn PWM Control register (CT16Bn_PWMCTRL) (n=0).......................................... 145 11.10.12 CT16Bn PWM Control register (CT16Bn_PWMCTRL) (n=1,5)....................................... 147 11.10.13 CT16Bn Timer Raw Interrupt Status register (CT16Bn_RIS) (n=0,1,5) ........................... 148 11.10.14 CT16Bn Timer Interrupt Clear register (CT16Bn_IC) (n=0,1,5)...................................... 148

SONiX TECHNOLOGY CO., LTD Page 8 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

11.10.15 CT16Bn Timer Match register 9 (CT16Bn_MR9) (n=0,1,5).............................................. 149 11.10.16 CT16Bn PWMmN Dead-band Period register (CT16Bn_PWMmNDB) (n=0).................. 149 11.10.17 CT16Bn PWM Load Mode Control register (CT16Bn_LOADCTRL) (n=0,1,5) ............... 149 11.10.18 CT16Bn DMA Mode register (CT16Bn_DMA) (n=0,1,5).................................................. 150 11.10.19 CT16Bn DMA MRm Alias register 1 (CT16Bn_DMAMRA1) (n=0,1,5)............................ 150 11.10.20 CT16Bn DMA MRm Alias register 1 (CT16Bn_DMAMRA2) (n=0,1,5)............................ 150 11.10.21 CT16Bn Break Function Control register (CT16Bn_BRKCTRL) (n=0)............................ 151

WATCHDOG TIMER (WDT)........................................................................................................ 152

12.1 OVERVIEW................................................................................................................................... 152 12.2 BLOCK DIAGRAM....................................................................................................................... 153 12.3 WDT REGISTERS......................................................................................................................... 154

12.3.1 Watchdog Configuration register (WDT_CFG)..................................................................... 154 12.3.2 Watchdog Timer Constant register (WDT_TC)...................................................................... 154 12.3.3 Watchdog Feed register (WDT_FEED) ................................................................................. 155

REAL-TIME CLOCK (RTC) ......................................................................................................... 156

13.1 OVERVIEW................................................................................................................................... 156 13.2 FEATURES.................................................................................................................................... 156 13.3 FUNCTIONAL DESCRIPTION.................................................................................................... 156

13.3.1 INTRODUCTION................................................................................................................... 156 13.3.2 RTC FLAG ASSERTION ........................................................................................................ 156 13.3.3 RTC OPERATION.................................................................................................................. 156

13.4 BLOCK DIAGRAM....................................................................................................................... 157 13.5 RTC REGISTERS .......................................................................................................................... 158 13.5.1 RTC Control register (RTC_CTRL) ....................................................................................... 158 13.5.2 RTC Clock Source Select register (RTC_CLKS).................................................................... 158 13.5.3 RTC Interrupt Enable register (RTC_IE)............................................................................... 158 13.5.4 RTC Raw Interrupt Status register (RTC_RIS) ...................................................................... 158 13.5.5 RTC Interrupt Clear register (RTC_IC)................................................................................. 159 13.5.6 RTC Second Counter Reload Value register (RTC_SECCNTV)............................................ 159 13.5.7 RTC Second Count register (RTC_SECCNT) ........................................................................ 159

SPI...................................................................................................................................................... 160

14.1 OVERVIEW................................................................................................................................... 160 14.2 FEATURES.................................................................................................................................... 160 14.3 PIN DESCRIPTION....................................................................................................................... 160 14.4 INTERFACE DESCRIPTION ....................................................................................................... 161

14.4.1 SPI .......................................................................................................................................... 161 14.4.2 COMMUNICATION FLOW................................................................................................... 162 14.4.2.1 SINGLE-FRAME........................................................................................................... 162

SONiX TECHNOLOGY CO., LTD Page 9 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

14.4.2.2 MULTI-FRAME ............................................................................................................ 162 14.5 AUTO-SEL..................................................................................................................................... 163 14.6 ARGB MODE................................................................................................................................. 163

14.6.1 Master Mode MOSI ................................................................................................................ 163 14.6.2 DMA Transfer......................................................................................................................... 163 14.7 DMA MODE .................................................................................................................................. 164 14.7.1 SPI DMA TX Mode ................................................................................................................. 165 14.7.2 SPI DMA RX Mode................................................................................................................. 166 14.7.3 SPI DMA Full-Duplex Mode .................................................................................................. 167 14.7.4 SPI DMA Configuration Recommendations........................................................................... 167 14.7.4.1 SPI DMA TX Only......................................................................................................... 168 14.7.4.2 SPI DMA RX Only......................................................................................................... 169 14.7.4.3 SPI Full-duplex DMA Mode (DL = 16 bit)................................................................. 170 14.7.4.4 SPI Full-duplex DMA Mode (DL = 8 bit)...................................................................... 171 14.8 TIMING CHARACTERISTICS..................................................................................................... 172 14.8.1 MASTER MODE..................................................................................................................... 172 14.8.2 SLAVE MODE........................................................................................................................ 172 14.9 SPI REGISTERS ............................................................................................................................ 173 14.9.1 SPI n Control register 0 (SPIn_CTRL0) (n=0) ...................................................................... 173 14.9.2 SPI n Control register 1 (SPIn_CTRL1) (n=0) ...................................................................... 174 14.9.3 SPI n Clock Divider register (SPIn_CLKDIV) (n=0) ............................................................ 174 14.9.4 SPI n Status register (SPIn_STAT) (n=0)............................................................................... 174 14.9.5 SPI n Interrupt Enable register (SPIn_IE) (n=0)................................................................... 175 14.9.6 SPI n Raw Interrupt Status register (SPIn_RIS) (n=0).......................................................... 175 14.9.7 SPI n Interrupt Clear register (SPIn_IC) (n=0)..................................................................... 176 14.9.8 SPI n Data register (SPIn_DATA) (n=0) ............................................................................... 176 14.9.9 SPI n Data Fetch register (SPIn_DF) (n=0).......................................................................... 176 14.9.10 SPI n FIFO threshold register (SPIn_FIFO_TH) (n=0).................................................... 176 14.9.11 SPI n DMA Mode register (SPIn_DMA) (n=0,1)............................................................... 177 14.9.12 SPI n ARGB CTRL register (SPIn_DMAHTCNT) (n=0,1)................................................ 177

I2C...................................................................................................................................................... 178

15.1 OVERVIEW................................................................................................................................... 178 15.2 FEATURES.................................................................................................................................... 178 15.3 PIN DESCRIPTION....................................................................................................................... 178 15.4 I2C PROTOCOL ............................................................................................................................ 179

15.4.1 7-BIT ADDRESSING MODES ............................................................................................... 179 15.4.1.1 MASTER TRANSMITTER MODE .............................................................................. 179 15.4.1.2 MASTER RECEIVER MODE....................................................................................... 180 15.4.1.3 SLAVE TRANSMITTER MODE.................................................................................. 180

SONiX TECHNOLOGY CO., LTD Page 10 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

15.4.1.4 SLAVE RECEIVER MODE.......................................................................................... 180 15.4.2 10-BIT ADDRESSING MODES ............................................................................................. 180 15.4.2.1 MASTER TRANSMITTER MODE .............................................................................. 180 15.4.2.2 MASTER RECEIVER MODE....................................................................................... 180 15.5 ARBITRATION ............................................................................................................................. 181 15.6 CLOCK STRETCHING................................................................................................................. 181 15.7 GENERAL CALL ADDRESS ....................................................................................................... 181 15.8 TIMING CHARACTERISTICS..................................................................................................... 182 15.8.1 MASTER TRANSMITTER MODE.......................................................................................... 182 15.8.2 SLAVE TRANSMITTER MODE ............................................................................................. 182 15.8.3 I2C TX_BIT Changing Timing ............................................................................................... 183 15.9 DMA MODE .................................................................................................................................... 184 15.9.1 I2C DMA Master TX Mode .................................................................................................... 184 15.9.2 I2C DMA Master RX Mode .................................................................................................... 185 15.9.3 I2C DMA Slave TX Mode ....................................................................................................... 187 15.9.4 I2C DMA Slave RX Mode....................................................................................................... 188 15.9.5 I2C DMA Configuration Recommendations........................................................................... 189 15.9.5.1 I2C TX DMA.................................................................................................................. 189 15.9.5.2 I2C RX DMA ................................................................................................................. 189 15.10 I2C REGISTERS ........................................................................................................................ 190 15.10.1 I2C n Control register (I2Cn_CTRL) (n=0)....................................................................... 190 15.10.2 I2C n Status register (I2Cn_STAT) (n=0).......................................................................... 191 15.10.3 I2C n TX Data register (I2Cn_TXDATA) (n=0) ................................................................ 192 15.10.4 I2C n RX Data register (I2Cn_RXDATA) (n=0)................................................................ 192 15.10.5 I2C n Slave Address 0 register (I2Cn_SLVADDR0) (n=0)................................................ 192 15.10.6 I2C n Slave Address 1~3 register (I2Cn_SLVADDR1~3) (n=0) ....................................... 192 15.10.7 I2C n SCL High Time register (I2Cn_SCLHT) (n=0)........................................................ 193 15.10.8 I2C n SCL Low Time register (I2Cn_SCLLT) (n=0).......................................................... 193 15.10.9 I2C n Timeout Control register (I2Cn_TOCTRL) (n=0) ................................................... 193 15.10.1 I2C n DMA Mode register (I2Cn_DMA) (n=0) ................................................................. 193

UNIVERSAL ASYNCHRONOUS RECEIVER AND TRANSMITTER (UART).................... 194

16.1 OVERVIEW................................................................................................................................... 194 16.2 FEATURES.................................................................................................................................... 194 16.3 PIN DESCRIPTION....................................................................................................................... 194 16.4 BLOCK DIAGRAM....................................................................................................................... 195 16.5 BAUD RATE CALCULATION .................................................................................................... 196 16.6 DMA MODE .................................................................................................................................. 198

16.6.1 UART DMA TX Mode............................................................................................................. 198 16.6.2 UART DMA RX Mode............................................................................................................. 199

SONiX TECHNOLOGY CO., LTD Page 11 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

16.7 UART REGISTERS ....................................................................................................................... 201 16.7.1 UART n Receiver Buffer register (UARTn_RB) (n=0,1)........................................................ 201 16.7.2 UART n Transmitter Holding register (UARTn_TH) (n=0,1)................................................ 201 16.7.3 UART n Divisor Latch LSB registers (UARTn_DLL) (n =0,1).............................................. 201 16.7.4 UART n Divisor Latch MSB register (UARTn_DLM) (n=0,1) .............................................. 202 16.7.5 UART n Interrupt Enable register (UARTn_IE) (n=0,1) ....................................................... 202 16.7.6 UART n Interrupt Identification register (UARTn_II) (n=0,1).............................................. 202 16.7.7 UART n FIFO Control register (UARTn_FIFOCTRL) (n=0,1)............................................. 203 16.7.8 UART n Line Control register (UARTn_LC) (n=0,1) ............................................................ 203 16.7.9 UART n Line Status register (UARTn_LS) (n=0,1)................................................................ 204 16.7.10 UART n Scratch Pad register (UARTn_SP) (n=0,1).......................................................... 205 16.7.11 UART n Fractional Divider register (UARTn_FD) (n=0,1).............................................. 205 16.7.12 UART n Control register (UARTn_CTRL) (n=0,1)............................................................ 205 16.7.13 UART n Half-duplex Enable register (UARTn_HDEN) (n=0,1) ....................................... 206 16.7.1 UART n DMA Mode register (UARTn_DMA) (n=0,1) .......................................................... 206

FOC.................................................................................................................................................... 207

17.1 OVERVIEW................................................................................................................................... 207 17.2 FEATURES.................................................................................................................................... 207 17.3 FUNCTION DESCRIPTIONS....................................................................................................... 207

17.3.1 Clarke and Inverse-Clarke Transformers............................................................................... 207 17.3.2 Park and Inverse-Park Transformers..................................................................................... 208 17.3.3 PI Controllers......................................................................................................................... 208 17.3.4 SVPWM Calculator ................................................................................................................ 208

17.4 FOC REGISTERS .......................................................................................................................... 209 17.4.1 FOC Control register (FOC_CTRL)...................................................................................... 209 17.4.2 FOC Control register 1 (FOC_CTRL1)................................................................................. 209 17.4.3 FOC Motor-A Phase Current register (FOC_IA).................................................................. 210 17.4.4 FOC Motor-B Phase Current register (FOC_IB).................................................................. 210 17.4.5 FOC Motor-C Phase Current register (FOC_IC).................................................................. 210 17.4.6 FOC Motor Stator Output Angle register (FOC_ANGLE) .................................................... 210 17.4.7 FOC Stator Flux Command register (FOC_FLX_CMD)....................................................... 210 17.4.8 FOC Motor Stator Flux Feedback register (FOC_FLX_FB) ................................................ 210 17.4.9 FOC Current D Feedforward register (FOC_ID_FF)........................................................... 211 17.4.10 FOC Rotor Speed Command register (FOC_SPD_CMD)................................................. 211 17.4.11 FOC Motor Rotor Speed Feedback register (FOC_SPD_FB)........................................... 211 17.4.12 FOC Q-axis Current Feedforward register (FOC_IQ_FF)............................................... 211 17.4.13 FOC D-axis Current Command register (FOC_ID_CMD) ............................................... 211 17.4.14 FOC D-axis Voltage Feedforward register (FOC_VD_FF).............................................. 211 17.4.15 FOC Q-axis Current Command register (FOC_IQ_CMD) ............................................... 212

SONiX TECHNOLOGY CO., LTD Page 12 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

17.4.16 FOC Q-axis Voltage Feedforward register (FOC_VQ_FF).............................................. 212 17.4.17 FOC SVPWM Output Ratio register (FOC_SVP_RATIO)................................................. 212 17.4.18 FOC Motor Alpha-axis Current register (FOC_IALFA) ................................................... 212 17.4.19 FOC Motor Beta-axis Current register (FOC_IBETA) ..................................................... 212 17.4.20 FOC Motor D-axis Current register (FOC_ID)................................................................. 212 17.4.21 FOC Motor Q-axis Current register (FOC_IQ)................................................................. 213 17.4.22 FOC Driver D-axis Output Voltage register (FOC_VD)................................................... 213 17.4.23 FOC Driver Q-axis Output Voltage register (FOC_VQ)................................................... 213 17.4.24 FOC Driver Alpha-axis Output Voltage register (FOC_VALFA)...................................... 213 17.4.25 FOC Driver Beta-axis Output Voltage register (FOC_VBETA)........................................ 213 17.4.26 FOC Driver A-phase Output Voltage register (FOC_VA)................................................. 213 17.4.27 FOC Driver B-phase Output Voltage register (FOC_VB)................................................. 214 17.4.28 FOC Driver C-phase Output Voltage register (FOC_VC) ................................................ 214 17.4.29 FOC SVPWM Maximum Value register (FOC_SVP_MAX).............................................. 214 17.4.30 FOC SVPWM Minimum Value register (FOC_SVP_MIN)................................................ 214 17.4.31 FOC SVPWM Delta Value register (FOC_SVP_DLTA).................................................... 214 17.4.32 FOC SVPWM A-phase Output Voltage register (FOC_SVP_A)........................................ 214 17.4.33 FOC SVPWM B-phase Output Voltage register (FOC_SVP_B)........................................ 215 17.4.34 FOC SVPWM C-phase Output Voltage register (FOC_SVP_C)....................................... 215 17.4.35 FOC Proportional Gain for Flux PI Controller register (FOC_FLX_PI_KP).................. 215 17.4.36 FOC Integral Gain for Flux PI Controller register (FOC_FLX_PI_KI)........................... 215 17.4.37 FOC Maximum Output Limit for Flux PI Controller register (FOC_FLX_PI_MAX)....... 215 17.4.38 FOC Minimum Output Limit for Flux PI Controller register (FOC_FLX_PI_MIN) ........ 215 17.4.57 FOC Proportional Gain for D-axis Current PI Controller register (FOC_ID_PI_KP) ... 219

ACC.................................................................................................................................................... 224

18.1 OVERVIEW................................................................................................................................... 224 18.4.1 ACC Hardware Calculation Accelerator Control register (ACC_CTRL) ............................. 225 18.4.2 ACC Arctangent Input register (ACC_ATAN_IN).................................................................. 225 18.4.3 ACC Arctangent Output register (ACC_ATAN_OUTPUT) ................................................... 225 18.4.4 ACC Dividend Input for Division register (ACC_DIV_DVD) ............................................... 226 18.4.5 ACC Divisor Input for Division register (ACC_DIV_DVS)................................................... 226 18.4.6 ACC Quotient Output for Division register (ACC_DIV_QUO)............................................. 226

18.5 ACC REMAINDER OUTPUT REGISTER (ACC_DIV_REM)............................................................... 226 18.6 ACC SQRT INPUT REGISTER (ACC_SQRT_IN)............................................................................. 226 18.7 ACC SQRT OUTPUT REGISTER (ACC_SQRT_OUT)..................................................................... 226

CYCLIC REDUNDANCY CHECK (CRC)................................................................................... 227

19.1 OVERVIEW................................................................................................................................... 227 19.2 FEATURES.................................................................................................................................... 227

SONiX TECHNOLOGY CO., LTD Page 13 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

19.3 CRC REGISTERS .......................................................................................................................... 228 19.3.1 CRC Control register (CRC_CTRL)....................................................................................... 228 19.3.2 CRC Data register (CRC_DATA)........................................................................................... 228

FLASH............................................................................................................................................... 229

20.1 OVERVIEW................................................................................................................................... 229 20.2 EMBEDDED FLASH MEMORY.................................................................................................. 229 20.3 FEATURES.................................................................................................................................... 229 20.4 ORGANIZATION.......................................................................................................................... 230 20.5 READ ............................................................................................................................................. 230 20.6 PROGRAM/ERASE....................................................................................................................... 230 20.7 EMBEDDED BOOT LOADER ..................................................................................................... 230 20.8 FLASH MEMORY CONTROLLER (FMC).................................................................................. 231

20.8.1 Code Security (CS) ................................................................................................................. 231 20.8.2 Program FLASH Memory ...................................................................................................... 232 20.8.3 Erase....................................................................................................................................... 232

20.8.3.1 Page Erase....................................................................................................................... 232 20.8.3.2 Mass Erase...................................................................................................................... 232 20.9 READ PROTECTION.................................................................................................................... 232 20.10 HW CHECKSUM....................................................................................................................... 232 20.11 FMC REGISTERS...................................................................................................................... 233 20.11.1 Flash Low Power Control register (FLASH_LPCTRL)..................................................... 233 20.11.2 Flash Status register (FLASH_STATUS) ........................................................................... 233 20.11.3 Flash Control register (FLASH_CTRL)............................................................................. 233 20.11.4 Flash Data register (FLASH_DATA)................................................................................. 234 20.11.5 Flash Address register (FLASH_ADDR) ........................................................................... 234 20.11.6 Flash Checksum register (FLASH_CHKSUM).................................................................. 234 20.11.7 Flash Write Protect register (FLASH_WP)........................................................................ 234

SERIAL-WIRE DEBUG (SWD)..................................................................................................... 235

21.1 OVERVIEW................................................................................................................................... 235 21.2 FEATURES.................................................................................................................................... 235 21.3 PIN DESCRIPTION....................................................................................................................... 235 21.4 DEBUG NOTE............................................................................................................................... 235

21.4.1 LIMITATIONS ........................................................................................................................ 235 21.4.2 DEBUG RECOVERY.............................................................................................................. 235 21.4.3 INTERNAL PULL-UP/DOWN RESISTORS on SWD PINS ................................................... 235

DEVELOPMENT TOOL ................................................................................................................ 236

22.1 SN-LINK-V3.................................................................................................................................. 237 22.2 SN32F400 STARTER-KIT ............................................................................................................ 238

SONiX TECHNOLOGY CO., LTD Page 14 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

ELECTRICAL CHARACTERISTIC............................................................................................ 239

23.1 ABSOLUTE MAXIMUM RATING.............................................................................................. 239 23.2 ELECTRICAL CHARACTERISTIC............................................................................................. 239 23.3 CHARACTERISTIC GRAPHS ..................................................................................................... 242

FLASH ROM PROGRAMMING PIN........................................................................................... 244

PACKAGE INFORMATION ......................................................................................................... 245

25.1 LQFP 48 PIN .................................................................................................................................. 245 25.2 QFN 32 PIN 4X4 ............................................................................................................................. 246 25.3 LQFP 32 PIN .................................................................................................................................. 247 25.4 QFN 24 PIN 4X4 ............................................................................................................................. 248

MARKING DEFINITION............................................................................................................... 249

26.1 MARKING INDETIFICATION SYSTEM.................................................................................... 249 26.2 INTRODUCTION.......................................................................................................................... 250 26.3 MARKING EXAMPLE ................................................................................................................. 250 26.4 DATECODE SYSTEM.................................................................................................................. 251

SONiX TECHNOLOGY CO., LTD Page 15 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

PRODUCT OVERVIEW

1.1 FEATURES

♦ Memory configuration ♦ Timer

32KB on-chip Flash programming memory. 3 16-bit timer support up-counting, down-counting, and 8KB SRAM. center-aligned mode.

4KB Boot ROM 4 sets inverse PWM with programmable dead-band PCLK up to 120MHz

♦ Operation Frequency up to 60MHz

♦ Working voltage 1.8V ~ 5.5V

♦ Interrupt sources

ARM® Cortex®-M0 built-in Nested Vectored Interrupt ♦ 16+6 CH 12-bit SAR ADC

Controller (NVIC). 16 external ADC input

1 internal battery measurement

♦ I/O pin configuration 4-level internal reference voltage source (VDD, 4.5V,  Up to 46 General Purpose I/O (GPIO) pins with 3V, 2V, 1.5V)

configurable pull-up/pull-down resistors. 22-frame ADB FIFO

GPIO pins can be used as edge and level sensitive Single/Continuous mode

interrupt sources. Single/Multi-channel auto conversion All IO with typical 30mA driving/sinking current.

♦ 3 OPA

♦ Programmable Watchdog Timer (WDT) 1X~32X PGA

Programmable watchdog frequency with watchdog

clock source and divider. ♦ 4 Rail to Rail Comparators

Internal 8-bit DAC reference voltage source

♦ System tick timer Programmable 255-level internal ref. voltage divider The 24-bit SysTick timer clock source is fixed to the 3 external negative inputs

system clock, and is intended to generate a fixed 10-ms 3 external positive inputs

interrupt.

♦ FOC

♦ Real-Time Clock (RTC) Flux, speed, current PI controllers Park/ipark transformers

♦ LVD with separate thresholds Clark/iclark transformers Reset: 2.1V/2.5V/2.9V/3.3V/3.7V/4.1V/4.5V for VDD SVPWM

Interrupt: 2.1V/2.5V/2.9V/3.3V/3.7V/4.1V/4.5V for VDD

♦ ACC

♦ Fcpu (Instruction cycle) Calculation accelerator

FCPU = FHCLK = FSYSCLK/1, F SYSCLK /2, F SYSCLK /4, …, Include arctangent, division, and square root. FSYSCLK /128.

♦ Interface

♦ Operating modes -One I2C controllers supporting I2C-bus specification  Normal, Sleep, Deep-sleep with multiple address recognition. -Two UART controllers with fractional baud rate

♦ Cyclic Redundancy Check (CRC) generation.

CRC-16 -One SPI controller.

CRC-16-CCCITT

CRC-32 ♦ System clocks

-External high clock: Crystal type 4MHz~25MHz

♦ 5-channel DMA controller -External low clock: Crystal type 32.768 KHz -Internal high clock: RC type 48MHz

♦ PFPA -Internal low clock: RC type 32 KHz -PLL allows CPU operation up to the maximum CPU

♦ Serial Wire Debug (SWD) rate without the need for a high-frequency crystal.  -Clock output function which can reflect the internal

♦ In-Circuit Programming (ICP) supported high/low RC oscillator, HCLK, PLL output, and  external low clock.

♦ Package (Chip form support)

LQFP48/32 pin

QFN32/24 pin

SONiX TECHNOLOGY CO., LTD Page 16 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

☞ Features Selection Table

| Chip | ROM (KB) | RAM (KB) | Fcpu. (Max MHz) | TIMER | UART | I2C | SPI | PWM | 12-bit ADC | CMP | OPA | GPIO with Wakeup | PKG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SN32F407F | 32 | 8 | 60 | 24-bit x 1 16-bit x 3 | 2 | 1 | 1 | 12+4 | 16+3 | 4 | 3 | 46 | LQFP48 |
| SN32F405F/J | 32 | 8 | 60 | 24-bit x 1 16-bit x 3 | 2 | 1 | 1 | 12+4 | 16+3 | 4 | 3 | 30 | QFN32/ LQFP32 |
| SN32F403J | 32 | 8 | 60 | 24-bit x 1 16-bit x 3 | 2 | 1 | 1 | 12+4 | 16+3 | 4 | 3 | 23 | QFN24 |

SONiX TECHNOLOGY CO., LTD Page 17 Version 1.1

1.2 SYSTEM BLOCK DIAGRAM

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

SWDIO SWCLK

DMA Controller TEST/DEBUG INTERFACE ARM

CORTEX-M0

M1 M0

Multilayer AHB Matrix

S3 S2 S1 S0

POWER

REGULATOR 1 VCORE

VDD 1.8V~5.5V

HXTALIN,  LXTALIN HXTALOUT,  LXTALOUT

CLOCK GENERATION

Clocks

SRAM 8KB

FLASH ROM 32KB

FLASH ROM 4KB (BOOT LOADER)

CLKOUT

ILRC 32KHz

SYS

IHRC 12MHz

AHB TO APB1

AHB TO APB0

Controls

POWER CONTROL/ SYSTEM FUNCTIONS

/RESET

URXD0 UTXD0 URXD1 UTXD1 SCL0

FMC

DMAC register UART 0

UART 1

I2C 0

APB1 BUS

BRIDGE

BRIDGE

WDT

RTC PMU CRC PFPA GPIO

LVD

P0.0~3, P0.5, P0.10~11 P1.0~3

P2.0~10

P3.0~2, P3.5~7,

P3.12~13

SDA0 SCK0

APB0 BUS

ADC AIN0~AIN15 CT16B0_PWM[3:0]

SEL0 MISO0 MOSI0 OP0P OP0N OP0O OP1P OP1N OP1O OP2P OP2N OP2O

SPI 0 OPA0 OPA1 OPA2 FOC ACC

16-bit TIMER 0 with PWM 16-bit TIMER 1 with PWM 16-bit TIMER 5 with PWM CMP 0

CMP 1

CMP 2

CMP 3

CT16B0_PWMN[3:0] CT16B0_CAP0

CT16B0_BRK

CT16B1_PWM[3:0] CT16B1_CAP0

CT16B5_PWM[3:0] CT16B5_CAP0

CM0P[2:0]

CM0N[2:0]

CM0O

CM1P[2:0]

CM1N[2:0]

CM1O

CM2P[2:0]

CM2N[2:0]

CM2O

CM3P[2:0]

CM3N[2:0]

CM3O

SONiX TECHNOLOGY CO., LTD Page 18 Version 1.1

1.3 CLOCK GENERATION BLOCK DIAGRAM

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

HCLK

AHB clock for AHB to APB bridge, to AHB matrix, to  Cortex-M0 FCLK, HCLK, and System Timer, to SYS,  and to PMU

CLKOUT

CLKOUT Prescaler

/1,2,4, ,128

CLKOUTSEL

/4

PLLCLKout HCLK

IHRC

ILRC

32.768KHz

IHRC  Prescaler /1 or /4

/8

IHRC ILRC

ILRC 256KHz IHRC 48MHz

SYS

ADCCLKEN

WDTCLKEN

I2CnCLKEN

n= 0

SPInCLKEN n= 0

/3

WDT

Clock Prescaler

/1, 2, 4, 8, 16, 32 I2Cn

Clock Prescaler /1, 2, 4, 8, 16

ADC_PCLK Max 16MHz WDT_PCLK I2Cn_PCLK SPIn_PCLK

ADC

clock source WDT

clock source I2Cn

clock source SPIn

clock source

AHB clock for ADC ADC

register block

AHB clock for WDT WDT

register block

AHB clock for I2Cn I2Cn

register block

AHB clock for SPIn SPIn

register block

32.768KHz

SYSCLK HCLK

AHB clock for UARTn

LXIN Low speed Crystal  oscillator

EHS Oscillator PLLCLKout

Prescaler /1 or /1.5 DIV1P5

AHB Prescaler /1,2,4, ,128 AHBCP

UARTnCLKEN

UARTn_PCLK

UARTn

clock source

UARTn

register block

LXOUT

32.768KHz

SYSCLKSEL

n=0,1

CMP_PCLK

CMP

AHB clock for CMP

XIN HXTAL

PLLCLKSEL

ILRC

PLL

PLL_VCO

CMPCLKEN clock source RTCCLKEN

CMP

register block

AHB clock for RTC

10MHz~25MHz

RTC_PCLK RTC RTC

XOUT

GPIO block FLASH block PFPA blcok

32.768KHz

RTCCLKSEL

AHB clock for GPIO

AHB clock for FLASH

AHB clock for PFPA

AHB clock for SRAM

32.768KHz

PLL_VCO PLL_VCO

CT16BnCLKEN

n=5

CT16BnCLKSEL n=5

CT16BnCLKEN

n= 0,1

CT16BnCLKSEL n=0,1

CT16Bn_PCLK CT16Bn_PCLK CRC_PCLK

clock source CT16Bn

clock source CT16Bn

clock source CRC

register block

AHB clock for CT16Bn CT16Bn

register block

AHB clock for CT16Bn CT16Bn

register block

AHB clock for CRC

SRAM block

clock source CRCCLKEN

CRC

register block

OPA_PCLK

OPA

AHB clock for OPA

OPACLKEN clock source

OPA

register block

FOC

FOC_PCLK

FOC

AHB clock for FOC

FOCCLKEN

Clock Prescaler /1,2,4,8

clock source Max 9MHz

FOC

register block

ACC_PCLK

ACC

AHB clock for ACC

ACCCLKEN clock source

ACC

register block

SONiX TECHNOLOGY CO., LTD Page 19 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

1.4 PIN ASSIGNMENT

SN32F407F (LQFP 48 pins)

VDD

VSS

P1.5

P1.4

P1.3/BOOT

P0.11

P0.10

P1.2

P1.1

P1.0

P1.12

P1.11

48 47 46 45 44 43 42 41 40 39 38 37

B1_CAP0/P3.8 1 ● 36 P1.10 B0_CAP0/P3.9 2 35 P1.9 LXIN/P3.10 3 34 P1.8 LXOUT/P3.11 4 33 P3.6/SWCLK

XIN/P3.12 5

32 P3.5/SWDIO

XOUT/P3.13 6 31 P3.2

SN32F407F

RESET/P3.7 7 30 P3.1

B1_PWM0/CM0N0/AIN10/P2.10 8 29 P3.0 P0.4 9 28 P0.0

B5_CAP0/B1_PWM1/CM2N2/AIN14/P0.5 10 27 P0.1/CM2P2 B1_PWM2/B1_PWM0/CM0N2/AIN0/P2.0 11 26 P0.2 B1_PWM3/OP1N/CM0P0/AIN1/P2.1 12 25 P0.3/CM2P0 13 14 15 16 17 18 19 20 21 22 23 24

CM1N2/AIN2/P2.2 CM1P0/AIN3/P2.3 CM1N0/AIN4/P2.4 CM1N1/AIN5/P2.5 CM1P1/AIN6/P2.6 CM2N1/AIN7/P2.7 CM2P1/AIN8/P2.8 AIN9/P2.9 B1_PWM2/P0.6

CM1P2/B1_PWM3/P0.7 P1.6 P1.7

✴ Note: SONiX provide Boot loader to check the status of P1.3 (BOOT pin) during boot procedure. If BOOT pin is Low during Boot procedure, MCU will execute code in Boot loader instead of User code. We strongly recommended NOT using BOOT pin as output pin to drive the LED, otherwise, the BOOT pin status may be low during boot procedure.

SONiX TECHNOLOGY CO., LTD Page 20 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

SN32F405J/F (QFN 32 pins 4x4/LQFP 32 pins)

VSS P1.3/BOOT

P0.11

P0.10/CM3N1

P1.2/AIN13/CM3P1

P1.1/AIN12 /CM3N0

P1.0/AIN11/CM3P0

P3.6/SWCLK

32 31 30 29 28 27 26 25

VDD 1 ● 24 P3.5/SWDIO XIN/P3.12 2 23 P3.2

XOUT/P3.13 3

22 P3.1

RESET/P3.7 4 21 P3.0/AIN15/CLKOUT

SN32F405J/F

B1_PWM0/CM0N0/AIN10/P2.10 5 20 P0.0 B5_CAP0/B1_PWM1/CM2N2/AIN14/P0.5 6 19 P0.1/CM2P2 B1_PWM2/B1_PWM0/CM0N2/AIN0/P2.0 7 18 P0.2

B1_PWM3/OP1N/CM0P0/AIN1/P2.1 8 17 P0.3/CM2P0 9 10 11 12 13 14 15 16

OP1P/CM3P2/CM1N2/AIN2/P2.2 OP0N/CM3N2/CM1P0/AIN3/P2.3

OP0P/CM2N0/CM1N0/ CM0N1/AIN4/P2.4 OP0O/CM1N1/CM0P1/AIN5/P2.5 OP1O/CM1P1/AIN6/P2.6 OP2P/CM2N1/AIN7/P2.7 OP2O/CM2P1/AIN8/P2.8 OP2N/CM0P2/AIN9/P2.9

✴ Note: 1. The pins which are not pin-out shall be set correctly to decrease power consumption in low power modes. Strongly recommended to set these pins as input pull-up. 2. SONiX provide Boot loader to check the status of P1.3 (BOOT pin) during boot procedure. If BOOT pin is Low during Boot procedure, MCU will execute code in Boot loader instead of User code. We strongly recommended NOT using BOOT pin as output pin to drive the LED, otherwise, the BOOT pin status may be low during boot procedure.

SONiX TECHNOLOGY CO., LTD Page 21 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

SN32F403J (QFN 24 pins 4x4 pins)

P1.3/BOOT

P0.11

P0.10/CM3N1

P1.2/AIN13/CM3P1

P1.1/AIN12/CM3N0

P1.0/AIN11/CM3P0

24 23 22 21 20 19

VDD 1 ● 18 P3.6/SWCLK

OP0N/CM3N2/CM1P0/AIN3/P2.3 2

17 P3.5/SWDIO

OP1P/CM3P2/CM1N2/AIN2/P2.2 3 16 P3.2/B0_PWM3

SN32F403J

RESET/P3.7 4 15 P3.1/B0_PWM3N

OP1N/CM0P0/AIN1/P2.1 5 14 P0.0/B0_PWM1N

B1_PWM2/B1_PWM0/CM0N2/AIN0/P2.0 6 VSS 13 P0.1/B0_PWM1/CM2P2

7 8 9 10 11 12

OP0P/CM2N0/CM1N0/CM0N1/AIN4/P2.4 OP2P/CM2N1/AIN7/P2.7 OP2O/CM2P1/AIN8/P2.8 OP2N/CM0P2/AIN9/P2.9 B0_PWM0N/CM2P0/P0.3 B0_PWM0/P0.2

✴ Note: 1. The pins which are not pin-out shall be set correctly to decrease power consumption in low power modes. Strongly recommended to set these pins as input pull-up. 2. SONiX provide Boot loader to check the status of P1.3 (BOOT pin) during boot procedure. If BOOT pin is Low during Boot procedure, MCU will execute code in Boot loader instead of User code. We strongly recommended NOT using BOOT pin as output pin to drive the LED, otherwise, the BOOT pin status may be low during boot procedure.

SONiX TECHNOLOGY CO., LTD Page 22 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

1.5 PIN ALLOCATION TABLE

| Port | UART | SPI | I2C | CT16 | CMP | ADC | OPA | Other |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P0.0 |  | SCK0 |  | B0_PWM0 B0_PWM1N B0_PWM2 B0_PWM3N B5_PWM3 |  |  |  |  |
| P0.1 |  | SEL0 |  | B0_PWM0N B0_PWM1 B0_PWM2N B0_PWM3 B0_PWM3N B5_PWM2 | CM2P2 |  |  |  |
| P0.2 |  | MISO0 |  | B0_PWM0 B0_PWM1N B0_PWM3N B5_PWM1 |  |  |  |  |
| P0.3 |  | MOSI0 |  | B0_PWM0N B0_PWM3 B5_PWM0 | CM2P0 CM2O |  |  |  |
| P0.4 |  |  |  |  |  |  |  |  |
| P0.5 |  |  |  | B0_PWM3 B0_BRK B1_PWM1 B5_CAP0 | CM2N2 | AIN14 |  |  |
| P0.6 |  |  | SCL0 | B1_PWM2 |  |  |  |  |
| P0.7 |  |  | SDA0 | B1_PWM3 | CM1P2 |  |  |  |
| P0.10 | UTXD0 |  | SCL0 | B0_BRK B1_PWM2 | CM3N1 |  |  |  |
| P0.11 | URXD0 |  | SDA0 | B1_PWM3 |  |  |  |  |
| P1.0 | UTXD1 | SCK0 |  | B5_PWM2 | CM2O CM3P0 | AIN11 |  |  |
| P1.1 | URXD1 | MISO0 | SCL0 | B0_PWM0N B1_PWM0 B5_PWM0 | CM3N0 | AIN12 |  |  |
| P1.2 |  | MOSI0 | SDA0 | B1_PWM1 B5_PWM1 | CM3P1 | AIN13 |  |  |
| P1.3 |  |  |  | B5_PWM3 | CM3O |  |  | BOOT |
| P1.4 |  |  | SCL0 |  |  |  |  |  |
| P1.5 |  |  | SDA0 |  |  |  |  |  |
| P1.6 |  |  |  |  | CM0O |  |  |  |
| P1.7 |  | SEL0 |  |  |  |  |  |  |
| P1.8 | URXD1 | SEL0 |  |  |  |  |  |  |
| P1.9 | UTXD1 |  |  |  |  |  |  |  |
| P1.10 |  |  |  |  | CM1O |  |  |  |
| P1.11 |  |  |  |  | CM2O |  |  |  |
| P1.12 |  |  |  | B0_BRK B1_PWM0 |  |  |  |  |
| P2.0 | URXD0 |  |  | B1_PWM0 B1_PWM2 | CM0N2 CM0O | AIN0 |  |  |

SONiX TECHNOLOGY CO., LTD Page 23 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

| P2.1 | UTXD0 |  |  | B1_PWM3 B5_PWM3 | CM0P0 | AIN1 | OP1N |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P2.2 | UTXD1 |  |  | B5_PWM2 | CM1N2 CM3P2 CM1O | AIN2 | OP1P |  |
| P2.3 | URXD1 |  |  | B5_PWM1 | CM1P0 CM3N2 | AIN3 | OP0N |  |
| P2.4 |  | SEL0 |  | B0_PWM1N B1_PWM1 B5_PWM0 | CM0N1 CM1N0 CM2N0 | AIN4 | OP0P |  |
| P2.5 |  | SCK0 |  | B0_PWM1 | CM0P1 CM1N1 | AIN5 | OP0O |  |
| P2.6 |  | MISO0 |  | B0_PWM0 B0_PWM2N B5_PWM0 | CM0O CM1P1 | AIN6 | OP1O |  |
| P2.7 | UTXD1 | MOSI0 |  | B0_PWM0N B0_PWM2 B1_PWM2 B5_PWM1 | CM1O CM2N1 | AIN7 | OP2P |  |
| P2.8 | URXD1 |  |  | B0_PWM1N B0_PWM2 B0_PWM2N B1_PWM1 B5_PWM2 | CM2P1 CM3O | AIN8 | OP2O |  |
| P2.9 | UTXD0 |  |  | B0_PWM1 B0_PWM2N B1_PWM3 B5_PWM3 | CM0P2 | AIN9 | OP2N |  |
| P2.10 | URXD0 |  |  | B0_BRK B1_PWM0 | CM0N0 | AIN10 |  |  |
| P3.0 |  | SCK0 |  | B0_PWM0 | CM0O CM3O | AIN15 |  | CLKOUT |
| P3.1 | UTXD0 | MISO0 |  | B0_PWM1 B0_PWM3N | CM1O |  |  |  |
| P3.2 | URXD0 | MOSI0 |  | B0_PWM2 B0_PWM3 |  |  |  |  |
| P3.5 |  |  |  |  |  |  |  | SWDIO |
| P3.6 |  |  |  |  |  |  |  | SWCLK |
| P3.7 |  |  |  |  |  |  |  | RESET |
| P3.8 |  |  |  | B1_CAP0 | CM2O |  |  |  |
| P3.9 |  |  |  | B0_CAP0 |  |  |  |  |
| P3.10 |  |  |  |  |  |  |  | LXIN |
| P3.11 |  |  |  |  |  |  |  | LXOUT |
| P3.12 |  |  |  |  | CM3O |  |  | XIN |
| P3.13 |  |  |  |  |  |  |  | XOUT |

SONiX TECHNOLOGY CO., LTD Page 24 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

1.6 PIN DESCRIPTIONS

| Pad NAME | TYPE | DESCRIPTION |
| --- | --- | --- |
| VDD, VSS | P | Power supply input pins for digital circuit. |
| P0.0 | I/O | P0.0 — General-purpose digital input/output pin. |
| P0.1/CM2P2 | I/O | P0.1 — General-purpose digital input/output pin. |
|  | I | CM2P2 — Positive input pin 2 of comparator 2. |
| P0.2 | I/O | P0.2 — General-purpose digital input/output pin. |
| P0.3/CM2P0 | I/O | P0.3 — General-purpose digital input/output pin. |
|  | I | CM2P0 — Positive input pin 0 of comparator 2. |
| P0.4 | I/O | P0.4 — General-purpose digital input/output pin. |
| P0.5/AIN14/CM2N2/ CT16B5_CAP0/CT16B0_BRK | I/O | P0.5 — General-purpose digital input/output pin. |
|  | I | AIN14 — ADC channel input 14. |
|  |  | CM2N2 — Negative input pin 2 of comparator 2. |
|  |  | CT16B5_CAP0 — Capture input 0 for CT16B5. |
|  |  | CT16B0_BRK — Break input for CT16B0. |
| P0.6 | I/O | P0.6 — General purpose digital input/output pin. |
| P0.7/CM1P2 | I/O | P0.7 — General purpose digital input/output pin. |
|  | I | CM1P2 — Positive input pin 2 of comparator 1. |
| P0.10/CM3N1/CT16B0_BRK | I/O | P0.10 — General-purpose digital input/output pin. |
|  | I | LXIN — External low-speed X’tal input pin. |
|  |  | CM3N1 — Negative input pin 1 of comparator 3. |
|  |  | CT16B0_BRK — Break input for CT16B0. |
| P0.11 | I/O | P0.11 — General-purpose digital input/output pin. |
| P1.0/AIN11/CM3P0 | I/O | P1.0 — General-purpose digital input/output pin. |
|  | I | AIN11 — ADC channel input 11. |
|  |  | CM3P0 — Positive input pin 0 of comparator 3. |
| P1.1/AIN12/CM3N0 | I/O | P1.1 — General-purpose digital input/output pin. |
|  | I | AIN12 — ADC channel input 12. |
|  |  | CM3N0 — Negative input pin 0 of comparator 3. |
| P1.2/AIN13/CM3P1 | I/O | P1.2 — General-purpose digital input/output pin. |
|  | I | AIN13 — ADC channel input 13. |
|  |  | CM3P1 — Positive input pin 1 of comparator 3. |
| P1.3~P1.11 | I/O | P1.3~P1.11 — General-purpose digital input/output pin. |

SONiX TECHNOLOGY CO., LTD Page 25 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

| P1.12/CT16B0_BRK | I/O | P1.12 — General-purpose digital input/output pin. |
| --- | --- | --- |
|  | I | CT16B0_BRK — Break input for CT16B0. |
| P2.0/AIN0/CM0N2 | I/O | P2.0 — General-purpose digital input/output pin. |
|  | I | AIN0 — ADC channel input 0. |
|  |  | CM0N2 — Negative input pin 2 of comparator 0. |
| P2.1/AIN1/CM0P0/OP1N | I/O | P2.1 — General-purpose digital input/output pin. |
|  | I | AIN1 — ADC channel input 1. |
|  |  | CM0P0 — Positive input pin 0 of comparator 0. |
|  |  | OP1N — Negative input pin of OPA 1. |
| P2.2/AIN2/CM1N2/CM3P2/ OP1P | I/O | P2.2 — General-purpose digital input/output pin. |
|  | I | AIN2 — ADC channel input 2. |
|  |  | CM1N2 — Negative input pin 2 of comparator 1. |
|  |  | CM3P2 — Positive input pin 2 of comparator 3. |
|  |  | OP1P — Positive input pin of OPA 1. |
| P2.3/AIN3/CM1P0/CM3N2/ OP0N | I/O | P2.3 — General-purpose digital input/output pin. |
|  | I | AIN3 — ADC channel input 3. |
|  |  | CM1P0 — Positive input pin 0 of comparator 1. |
|  |  | CM3N2 — Negative input pin 2 of comparator 3. |
|  |  | OP0N — Negative input pin of OPA 0. |
| P2.4/AIN4/CM1N0/CM0N1/ CM2N0/OP0P | I/O | P2.4 — General-purpose digital input/output pin. |
|  | I | AIN4 — ADC channel input 4. |
|  |  | CM0N1 — Negative input pin 1 of comparator 0. |
|  |  | CM1N0 — Negative input pin 0 of comparator 1. |
|  |  | CM2N0 — Negative input pin 0 of comparator 2. |
|  |  | OP0P — Positive input pin of OPA 0. |
| P2.5/AIN5/CM1N1/CM0P1/ OP0O | I/O | P2.5 — General-purpose digital input/output pin. |
|  | I | AIN5 — ADC channel input 5. |
|  |  | CM0P1 — Positive input pin 1 of comparator 0. |
|  |  | CM1N1 — Negative input pin 1 of comparator 1. |
|  | O | OP0O — Output pin of OPA 0. |
| P2.6/AIN6/CM1P1/OP1O | I/O | P2.6 — General-purpose digital input/output pin. |
|  | I | AIN6 — ADC channel input 6. |
|  |  | CM1P1 — Positive input pin 1 of comparator 1. |
|  | O | OP1O — Output pin of OPA 1. |

SONiX TECHNOLOGY CO., LTD Page 26 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

| P2.7/AIN7/CM2N1/OP2P | I/O | P2.7 — General-purpose digital input/output pin. |
| --- | --- | --- |
|  | I | AIN7 — ADC channel input 7. |
|  |  | CM2N1 — Negative input pin 1 of comparator 2. |
|  |  | OP2P — Positive input pin of OPA 2. |
| P2.8/AIN8/CM2P1/OP2O | I/O | P2.8 — General-purpose digital input/output pin. |
|  | I | AIN8 — ADC channel input 8. |
|  |  | CM2P1 — Positive input pin 1 of comparator 2. |
|  | O | OP2O — Output pin of OPA 2. |
| P2.9/AIN9/CM0P2/OP2N | I/O | P2.9 — General-purpose digital input/output pin. |
|  | I | AIN9 — ADC channel input 9. |
|  |  | CM0P2 — Positive input pin 2 of comparator 0. |
|  |  | OP2N — Negative input pin of OPA 2. |
| P2.10/AIN10/CM0N0/ CT16B0_BRK | I/O | P2.10 — General-purpose digital input/output pin. |
|  | I | AIN10 — ADC channel input 10. |
|  |  | CM0N0 — Negative input pin 0 of comparator 0. |
|  |  | CT16B0_BRK — Break input for CT16B0. |
| P3.0/AIN15/CLKOUT | I/O | P3.0 — General-purpose digital input/output pin. |
|  | I | AIN15 — ADC channel input 15. |
|  | O | CLKOUT — Clockout pin |
| P3.1 | I/O | P3.1 — General-purpose digital input/output pin. |
| P3.2 | I/O | P3.2 — General-purpose digital input/output pin. |
| P3.5/SWDIO | I/O | P3.5 — General-purpose digital input/output pin. |
|  |  | SWDIO — Serial wire debug input/output. |
| P3.6/SWCLK | I/O | P3.6 — General-purpose digital input/output pin. |
|  | I | SWCLK — Serial wire clock. |
| P3.7/RESET | I/O | P3.7 — General-purpose digital input/output pin. |
|  | I | RESET — External Reset input. |
| P3.8~P3.9 | I/O | P3.8~P3.9 — General-purpose digital input/output pin. |
| P3.10/LXIN | I/O | P3.10 — General-purpose digital input/output pin. |
|  | I | LXIN — External low-speed X’tal input pin. |
| P3.11/LXOUT | I/O | P3.11 — General-purpose digital input/output pin. |
|  | O | LXOUT — External low-speed X’tal output pin. |
| P3.12/XIN | I/O | P3.12 — General-purpose digital input/output pin. |

SONiX TECHNOLOGY CO., LTD Page 27 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

|  | I | XIN — External high-speed X’tal input pin. |
| --- | --- | --- |
| P3.13/XOUT | I/O | P3.13 — General-purpose digital input/output pin. |
|  | O | XOUT — External high-speed X’tal output pin. |

Please refer to Peripheral Function Pin Assignment (PFPA) chapters for setting of each GPIOs.

SONiX TECHNOLOGY CO., LTD Page 28 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

● Normal Bi-direction I/O Pin.

1.7 PIN CIRCUIT DIAGRAMS VDD

RPU

Pin GPIOn_CFG I/O Input Bus

GPIOPn_MODE

RPD GPIOPn_MODE GPIOn_CFG

GPIOPn_MODE

Output Latch

I/O Output Bus

● Bi-direction I/O Pin Shared with Specific Digital Input Function, e.g., SPI, I2C…

VDD

Pin

GPIOPn_MODE

Specific Input Bus R Specific Input Function Control bit PU GPIOn_CFG I/O Input Bus

RPD GPIOPn_MODE GPIOn_CFG *.Specific Output Function Control bit GPIOPn_MODE

Output Latch

I/O Output Bus

*. Some specific functions switch I/O direction directly, not through GPIOn_MODE register.

SONiX TECHNOLOGY CO., LTD Page 29 Version 1.1

SN32F400 Series

● Bi-direction I/O Pin Shared with Specific Digital Output Function, e.g., SPI, I2C…  32-Bit Cortex-M0 Micro-Controller

VDD

RPU

Pin GPIOn_CFG I/O Input Bus GPIOPn_MODE RPD GPIOPn_MODE GPIOn_CFG

*.Specific Output Function Control bit

I/O Output Bus Specific Output Bus Specific Output Function Control bit *. Some specific functions switch I/O direction directly, not through GPIOn_MODE register. Output

GPIOPn_MODE

Latch

● Bi-direction I/O Pin Shared with Specific Analog Input Function, e.g., XIN, ADC…

GPIOPn_MODE

RPU

GPIOn_CFG I/O Input Bus

VDD

Pin

RPD GPIOPn_MODE GPIOn_CFG GPIOPn_MODE

Output Latch

I/O Output Bus

Analog IP Input Terminal

*. Specific Analog Function Control bit *. Some specific functions switch I/O direction directly, not through GPIOn_MODE register.

SONiX TECHNOLOGY CO., LTD Page 30 Version 1.1

● Bi-direction I/O Pin Shared with Specific Analog Output Function, e.g., XOUT…

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

GPIOPn_MODE

RPU

GPIOn_CFG I/O Input Bus

VDD

Pin

RPD GPIOPn_MODE GPIOn_CFG GPIOPn_MODE

Output Latch

I/O Output Bus

Analog IP Output Terminal

*. Specific Analog Function Control bit

*. Some specific functions switch I/O direction directly, not through GPIOn_MODE register.

SONiX TECHNOLOGY CO., LTD Page 31 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

1.8 PIN CHARACTERISTICS

| Port | Dir | ADC | CMP Input | OPA input | Open Drain | Pull-up Resistor | Pull-down Resistor | Schmitt trigger (VIH: 0.7*VDD VIL: 0.3*VDD) | Drive Current 30mA @VDD-0.5V | Sink Current 30mA @VSS+0.5V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P0.0 | I/O |  |  |  |  | V | V | V | V | V |
| P0.1 | I/O |  | V |  |  | V | V | V | V | V |
| P0.2 | I/O |  |  |  |  | V | V | V | V | V |
| P0.3 | I/O |  | V |  |  | V | V | V | V | V |
| P0.4 | I/O |  |  |  |  | V | V | V | V | V |
| P0.5 | I/O | V | V |  |  | V | V | V | V | V |
| P0.6 | I/O |  |  |  | V | V | V | V | V | V |
| P0.7 | I/O |  | V |  | V | V | V | V | V | V |
| P0.10 | I/O |  | V |  | V | V | V | V | V | V |
| P0.11 | I/O |  |  |  | V | V | V | V | V | V |
| P1.0 | I/O | V | V |  |  | V | V | V | V | V |
| P1.1 | I/O | V | V |  | V | V | V | V | V | V |
| P1.2 | I/O | V | V |  | V | V | V | V | V | V |
| P1.3 | I/O |  |  |  |  | V | V | V | V | V |
| P1.4 | I/O |  |  |  | V | V | V | V | V | V |
| P1.5 | I/O |  |  |  | V | V | V | V | V | V |
| P1.6 | I/O |  |  |  |  | V | V | V | V | V |
| P1.7 | I/O |  |  |  |  | V | V | V | V | V |
| P1.8 | I/O |  |  |  |  | V | V | V | V | V |
| P1.9 | I/O |  |  |  |  | V | V | V | V | V |
| P1.10 | I/O |  |  |  |  | V | V | V | V | V |
| P1.11 | I/O |  |  |  |  | V | V | V | V | V |
| P1.12 | I/O |  |  |  |  | V | V | V | V | V |
| P2.0 | I/O | V | V |  |  | V | V | V | V | V |
| P2.1 | I/O | V | V | V |  | V | V | V | V | V |
| P2.2 | I/O | V | V | V |  | V | V | V | V | V |
| P2.3 | I/O | V | V | V |  | V | V | V | V | V |
| P2.4 | I/O | V | V | V |  | V | V | V | V | V |
| P2.5 | I/O | V | V |  |  | V | V | V | V | V |
| P2.6 | I/O | V | V |  |  | V | V | V | V | V |
| P2.7 | I/O | V | V | V |  | V | V | V | V | V |
| P2.8 | I/O | V | V |  |  | V | V | V | V | V |
| P2.9 | I/O | V | V | V |  | V | V | V | V | V |
| P2.10 | I/O | V | V |  |  | V | V | V | V | V |
| P3.0 | I/O | V |  |  |  | V | V | V | V | V |
| P3.1 | I/O |  |  |  |  | V | V | V | V | V |
| P3.2 | I/O |  |  |  |  | V | V | V | V | V |
| P3.5 | I/O |  |  |  |  | V | V | V | V | V |
| P3.6 | I/O |  |  |  |  | V | V | V | V | V |
| P3.7 | I/O |  |  |  |  | V | V | V | V | V |
| P3.8 | I/O |  |  |  |  | V | V | V | V | V |
| P3.9 | I/O |  |  |  |  | V | V | V | V | V |
| P3.10 | I/O |  |  |  |  | V | V | V | V | V |
| P3.11 | I/O |  |  |  |  | V | V | V | V | V |
| P3.12 | I/O |  |  |  |  | V | V | V | V | V |
| P3.13 | I/O |  |  |  |  | V | V | V | V | V |

SONiX TECHNOLOGY CO., LTD Page 32 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

CENTRAL PROCESSOR UNIT (CPU)

2.1 MEMORY MAP

0xE 010 0000

NVIC Debug Control 0xE 000E000 Reserved

0 xFFFF FFFF

Reserved

0xE 000ED00 0xE 000F000 0xE 000 0000

DMA 0x4007 4000 Reserved 0x4008 0000 Reserved 0x4007 2000 Reserved

Reserved for External Device 0xE000 0000 Private Peripheral Bus 0xE010 0000 Reserved for External 0xA000 0000

FOC 0x4007 6000 ACC 0x4007 80000x4006 4000 0x4006 2000

0x4006 0000 FMC SYS0

SYS1 0x 4005 E000

Reserved for Peripheral 0x6000 0000

Reserved UART 1 Reserved

0x4005 8000

0x4005 6000 0x 4004C000 0x 4004A000

0x4008 0000

Peripheral

0x4004 4000 0x4004 6000 0x4004 8000 GPIO2 GPIO3

GPIO0 GPIO1

PFPA 0x4004 2000 Reserved

0x4003 8000 CRC 0x 4003A000

8 KB SRAM 0x4000 0000 Reserved

Reserved PMU

Reserved

0x4003 4000 0x4003 2000

0x2000 2000

OPA 0x 4002 C000 0x 4001C000 CMP 0x 4002A000

ADC

0x4002 8000 0x4002 6000

0x2000 0000

Reserved

0x 4001 E000 Reserved SPI0

4 KB Boot ROM 0x1FFF 1000

Reserved I2C0

0x 4001A000

0x1FFF 0000

Reserved

0x4001 0000 0x4001 2000 0x4001 4000 WDT 0x4001 6000 0x4001 8000 Reserved

UART0

Reserved

RTC

0x0000 8000 0x 4000A 000 CT16B5 0x 4000C000

0x0000 0000 32 KB on- chip FLASH

Reserved CT16B0 CT16B1

0x4000 0000 0x4000 2000 0x4000 4000

SONiX TECHNOLOGY CO., LTD Page 33 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

2.2 SYSTEM TICK TIMER

The SysTick timer is an integral part of the Cortex-M0. The SysTick timer is intended to generate a fixed 10-ms interrupt  for use by an operating system or other system management software.

Since the SysTick timer is a part of the Cortex-M0, it facilitates porting of software by providing a standard timer that is  available on Cortex-M0 based devices.

Refer to the Cortex-M0 User Guide for details.

2.2.1 OPERATION

The SysTick timer is a 24-bit timer that counts down to zero and generates an interrupt.

The block diagram of the SysTick timer:

The intent is to provide a fixed 10-ms time interval between interrupts. The system tick timer is enabled through the  SysTick control register. The system tick timer clock is fixed to the frequency of the system clock.

SYSTICK_CALIB

SYSTICK_LOAD

Load data Private Peripheral Bus

System Clock Ref. clock

SYSTICK_VAL 24-bit down counter

clock

10

ENABLE

CLKSOURCE (Fix to 1)

SYSTICK_CTRL

COUNTFLAG TICKINT

SysTick interrupt

When SysTick timer is enabled, the timer counts down from the current value (SYSTICK_VAL) to zero, reloads to the  value in the SysTick Reload Value Register (SYSTICK_LOAD) on the next clock edge, then decrements on subsequent  clocks. When the counter transitions to zero, the COUNTFLAG status bit is set to 1. The COUNTFLAG bit clears on  reads.

✴ Note: When the processor is halted for debugging the counter does not decrease.

SONiX TECHNOLOGY CO., LTD Page 34 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

2.2.2 SYSTICK USAGE HINTS AND TIPS

The interrupt controller clock updates the SysTick counter. Some implementations stop this clock signal for low power  mode. If this happens, the SysTick counter stops.

Ensure SW uses word accesses to access the SysTick registers.

The SysTick counter reload and current value are not initialized by HW. This means the correct initialization sequence  for the SysTick counter is:

1. Program the reload value in SYSTICK_LOAD register.

2. Clear the current value by writing any value to SYSTICK_VAL register.

3. Program the Control and Status (SYSTICK_CTRL) register.

2.2.3 SYSTICK REGISTERS

2.2.3.1 System Tick Timer Control and Status register (SYSTICK_CTRL) Address: 0xE000 E010 (Refer to Cortex-M0 Spec)

| Bit 31:17 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 16 | COUNTFLAG | This flag is set when the System Tick counter counts down to 0, and is cleared by reading this register. | R/W | 0 |
| 15:3 | Reserved |  | R | 0 |
| 2 | CLKSOURCE | Selects the SysTick timer clock source. 0: reference clock. 1: system clock. (Fixed) | R | 1 |
| 1 | TICKINT | System Tick interrupt enable. 0: Disable the System Tick interrupt 1: Enable the System Tick interrupt, the interrupt is generated when the System Tick counter counts down to 0. | R/W | 0 |
| 0 | ENABLE | System Tick counter enable. 0: Disable 1: Enable | R/W | 0 |

2.2.3.2 System Tick Timer Reload value register (SYSTICK_LOAD)

Address: 0xE000 E014 (Refer to Cortex-M0 Spec)

The RELOAD register is set to the value that will be loaded into the SysTick timer whenever it counts down to zero. This register is  set by software as part of timer initialization. The SYSTICK_CALIB register may be read and used as the value for RELOAD if the  CPU or external clock is running at the frequency intended for use with the SYSTICK_CALIB value.

The following example illustrates selecting the SysTick timer reload value to obtain a 10 ms time interval with the system clock set to  50 MHz

The SysTick clock = system clock = 50 MHz

RELOAD = (system tick clock frequency × 10 ms) −1 = (50 MHz × 10 ms) −1 = 0x0007A11F.

| Bit 31:24 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 23:0 | RELOAD | Value to load into the SYSTICK_VAL when the counter is enabled and when it reaches 0. | R/W | 0 |

SONiX TECHNOLOGY CO., LTD Page 35 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

2.2.3.3 System Tick Timer Current Value register (SYSTICK_VAL)

Address: 0xE000 E018 (Refer to Cortex-M0 Spec)

| Bit 31:24 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 23:0 | CURRENT | Reading this register returns the current value of the System Tick counter. Writing any value clears the System Tick counter and the COUNTFLAG bit in SYSTICK_CTRL. | R/W | 0x7E7F35 |

2.2.3.4 System Tick Timer Calibration Value register (SYSTICK_CALIB)

Address: 0xE000 E01C (Refer to Cortex-M0 Spec)

| Bit 31 | Name NOREF | Description Indicates the reference clock to M0 is provided or not. 1: No reference clock provided. | Attribute R | Reset 1 |
| --- | --- | --- | --- | --- |
| 30 | SKEW | Indicates whether the TENMS value is exact, an inexact TENMS value can affect the suitability of SysTick as a software real time clock. 0: TENMS value is exact 1: TENMS value is inexact, or not given. | R | 0 |
| 29:24 | Reserved |  | R | 0 |
| 23:0 | TENMS | Reload value for 10ms timing, subject to system clock skew errors. If the value reads as zero, the calibration value is not known. | R/W | 0xA71FF |

SONiX TECHNOLOGY CO., LTD Page 36 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

2.3 NESTED VECTORED INTERRUPT CONTROLLER (NVIC)

All interrupts including the core exceptions are managed by the NVIC. NVIC has the following Features: ⮚ The NVIC supports 32 vectored interrupts.

⮚ 4 programmable interrupt priority levels with hardware priority level masking.

⮚ Low-latency exception and interrupt handling.

⮚ Efficient processing of late arriving interrupts.

⮚ Implementation of System Control Registers

⮚ Software interrupt generation.

2.3.1 INTERRUPT AND EXCEPTION VECTORS

| Execution No. | Priority | Function | Description | Address Offset |
| --- | --- | --- | --- | --- |
| 0 | - | - | Reserved | 0x0000 0000 |
| 1 | -3 | Reset | Reset | 0x0000 0004 |
| 2 | -2 | NMI_Handler | Non maskable interrupt. | 0x0000 0008 |
| 3 | -1 | HardFault_Handler | All class of fault | 0x0000 000C |
| 4~10 | Reserved | Reserved | Reserved | - |
| 11 | Settable | SVCCalll |  | 0x0000 002C |
| 12~13 | Reserved | Reserved | Reserved | - |
| 14 | Settable | PendSV |  | 0x0000 0038 |
| 15 | Settable | SysTick |  | 0x0000 003C |
| 16 | Settable | IRQ0/NDTIRQ | NDT | 0x0000 0040 |
| 17 | Settable | IRQ1/DMA0IRQ | DMA0 | 0x0000 0044 |
| 18 | Settable | IRQ2/CMP3IRQ | CMP3 | 0x0000 0048 |
| 19 | Settable | IRQ3/ |  | 0x0000 004C |
| 20 | Settable | IRQ4/ |  | 0x0000 0050 |
| 21 | Settable | IRQ5/ |  | 0x0000 0054 |
| 22 | Settable | IRQ6/SPI0IRQ | SPI0 | 0x0000 0058 |
| 23 | Settable | IRQ7/ |  | 0x0000 005C |
| 24 | Settable | IRQ8/ |  | 0x0000 0060 |
| 25 | Settable | IRQ9 |  | 0x0000 0064 |
| 26 | Settable | IRQ10/I2C0IRQ | I2C0 | 0x0000 0068 |
| 27 | Settable | IRQ11/ |  | 0x0000 006C |
| 28 | Settable | IRQ12/CMP2IRQ | CMP2 | 0x0000 0070 |
| 29 | Settable | IRQ13/UART0IRQ | UART0 | 0x0000 0074 |
| 30 | Settable | IRQ14/UART1IRQ | UART1 | 0x0000 0078 |
| 31 | Settable | IRQ15/CT16B0IRQ | CT16B0 | 0x0000 007C |
| 32 | Settable | IRQ16/CT16B1IRQ | CT16B1 | 0x0000 0080 |

SONiX TECHNOLOGY CO., LTD Page 37 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

| 33 | Settable | IRQ17/SQRTIRQ | SQRT | 0x0000 0084 |
| --- | --- | --- | --- | --- |
| 34 | Settable | IRQ18/CMP1IRQ | CMP1 | 0x0000 0088 |
| 35 | Settable | IRQ19/ATANIRQ | ATAN | 0x0000 008C |
| 36 | Settable | IRQ20/DIVIRQ | DIV | 0x0000 0090 |
| 37 | Settable | IRQ21/CT16B5IRQ | CT16B5 | 0x0000 0094 |
| 38 | Settable | IRQ22/FOCIRQ | FOC | 0x0000 0098 |
| 39 | Settable | IRQ23/RTCIRQ | RTC | 0x0000 009C |
| 40 | Settable | IRQ24/ADCIRQ | ADC | 0x0000 00A0 |
| 41 | Settable | IRQ25/WDTIRQ | WDT | 0x0000 00A4 |
| 42 | Settable | IRQ26/LVDIRQ | LVD | 0x0000 00A8 |
| 43 | Settable | IRQ27/CMP0IRQ | CMP0 | 0x0000 00AC |
| 44 | Settable | IRQ28/P3IRQ | GPIO interrupt status of port 3 | 0x0000 00B0 |
| 45 | Settable | IRQ29/P2IRQ | GPIO interrupt status of port 2 | 0x0000 00B4 |
| 46 | Settable | IRQ30/P1IRQ | GPIO interrupt status of port 1 | 0x0000 00B8 |
| 47 | Settable | IRQ31/P0IRQ | GPIO interrupt status of port 0 | 0x0000 00BC |

2.3.2 NVIC REGISTERS

2.3.2.1 IRQ0~31 Interrupt Set-Enable Register (NVIC_ISER)

Address: 0xE000 E100 (Refer to Cortex-M0 Spec.)

The ISER enables interrupts, and shows the interrupts that are enabled.

| Bit 31:0 | Name SETENA[31:0] | Description Interrupt set-enable bits. Write🡪 0: No effect 1: Enable interrupt. Read🡪 0: Interrupt disabled 1: Interrupt enabled. | Attribute R/W | Reset 0 |
| --- | --- | --- | --- | --- |

2.3.2.2 IRQ0~31 Interrupt Clear-Enable Register (NVIC_ICER)

Address: 0xE000 E180 (Refer to Cortex-M0 Spec.)

The ICER disables interrupts, and shows the interrupts that are enabled.

| Bit 31:0 | Name CLRENA[31:0] | Description Interrupt clear-enable bits. Write🡪 0: No effect 1: Disable interrupt. Read🡪 0: Interrupt disabled 1: Interrupt enabled. | Attribute R/W | Reset 0 |
| --- | --- | --- | --- | --- |

2.3.2.3 IRQ0~31 Interrupt Set-Pending Register (NVIC_ISPR)

Address: 0xE000 E200 (Refer to Cortex-M0 Spec.)

SONiX TECHNOLOGY CO., LTD Page 38 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

The ISPR forces interrupts into the pending state, and shows the interrupts that are pending.

✴ Note: Writing 1 to the ISPR bit corresponding to 1. an interrupt that is pending has no effect 2. a disabled interrupt sets the state of that interrupt to pending.

| Bit 31:0 | Name SETPEND[31:0] | Description Interrupt set-pending bits. Write🡪 0: No effect 1: Change interrupt state to pending Read🡪 0: Interrupt is not pending 1: Interrupt is pending | Attribute R/W | Reset 0 |
| --- | --- | --- | --- | --- |

2.3.2.4 IRQ0~31 Interrupt Clear-Pending Register (NVIC_ICPR)

Address: 0xE000 E280 (Refer to Cortex-M0 Spec.)

The ICPR removes the pending state from interrupts, and shows the interrupts that are pending.

✴ Note: Writing 1 to an ICPR bit does not affect the active state of the corresponding interrupt.

| Bit 31:0 | Name CLRPEND[31:0] | Description Interrupt clear-pending bits. Write🡪 0: No effect 1: Removes pending state of an interrupt Read🡪 0: Interrupt is not pending 1: Interrupt is pending | Attribute R/W | Reset 0 |
| --- | --- | --- | --- | --- |

2.3.2.5 IRQ0~31 Interrupt Priority Register (NVIC_IPRn) (n=0~7)

Address: 0xE000 E400 + 0x4 * n (Refer to Cortex-M0 Spec.)

The interrupt priority registers provide an 8-bit priority field for each interrupt, and each register holds four priority fields.  This means the number of registers is implementation-defined, and corresponds to the number of implemented interrupts.

| Bit 31:24 | Name PRI_(4*n+3) | Description Each priority field holds a priority value, 0-192. The lower the value, the greater the priority of the corresponding interrupt. The processor implements only bits[31:30] of each field, bits [29:24] read as zero and ignore writes. This means writing 255 to a priority register saves value 192 to the register. | Attribute R/W | Reset 0 |
| --- | --- | --- | --- | --- |
| 23:16 | PRI_(4*n+2) | Each priority field holds a priority value, 0-192. The lower the value, the greater the priority of the corresponding interrupt. The processor implements only bits[23:22] of each field, bits [21:16] read as zero and ignore writes. This means writing 255 to a priority register saves value 192 to the register. | R/W | 0 |
| 15:8 | PRI_(4*n+1) | Each priority field holds a priority value, 0-192. The lower the value, the greater the priority of the corresponding interrupt. The processor implements only bits[15:14] of each field, bits [13:8] read as zero and ignore writes. This means writing 255 to a priority register saves value 192 to the register. | R/W | 0 |
| 7:0 | PRI_4*n | Each priority field holds a priority value, 0-192. The lower the value, the greater the priority of the corresponding interrupt. The processor implements only bits[7:6] of each field, bits [5:0] read as zero and ignore writes. This means writing 255 to a priority register saves value 192 to the register. | R/W | 0 |

2.4 APPLICATION INTERRUPT AND RESET CONTROL (AIRC)

Address: 0xE000 ED0C (Refer to Cortex-M0 Spec)

The entire MCU, including the core, can be reset by SW by setting the SYSRESREQ bit in the AIRC register in Cortex-

SONiX TECHNOLOGY CO., LTD Page 39 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

M0 spec.

✴ Note: To write to this register, user must write 0x05FA to the VECTKEY field at the same time, otherwise the processor ignores the write.

| Bit 31:16 | Name VECTKEY | Description Register key. Read as unknown. Write 0x05FA to VECTKEY, otherwise the write is ignored. | Attribute R/W | Reset 0 |
| --- | --- | --- | --- | --- |
| 15 | ENDIANESS | Data endianness implemented 0: Little-endian 1: Big-endian | R | 0 |
| 14:3 | Reserved |  | R | 0 |
| 2 | SYSRESETREQ | System reset request. This bit read as 0. 0: No effect 1: Requests a system level reset. | W | 0 |
| 1 | VECTCLRACTIVE | Reserved for debug use. This bit read as 0. When writing to the register you must write 0 to this bit, otherwise behavior is Unpredictable. | W | 0 |
| 0 | Reserved |  | R | 0 |

SONiX TECHNOLOGY CO., LTD Page 40 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

2.5 CODE OPTION TABLE

Address: 0x1FFF 2000

| Bit 31:16 | Name Code Security[15:0] | Description Code Security 0xFFFF: CS0 0x5A5A: CS1 0xA5A5: CS2 | Attribute R/W | Reset 0xFFFF |
| --- | --- | --- | --- | --- |
| 15:3 | Reserved |  | R/W | 0x1FFF |
| 2 | BOOTPINEN | Boot Pin enable 0: Disable 1: Enable | R/W | 1 |
| 1 | EXTRSTHWDIS | External reset HW disable bit 0: Enable (HW enables, FW can NOT control with SYS0_EXRSTCTRL) 1: Disable (FW can control with SYS0_EXRSTCTRL) | R/W | 1 |
| 0 | BLEN | Boot loader enable 0: Disable 1: Enable | R/W | 1 |

2.6 UNIQUE NUMBER

The unique number is an 8-byte unique device serial number of each IC. In other words, the unique number is different  and discontinuous for each IC.

Address: 0x1FFF 2DF0

| Bit 31:0 | Name L4BYTE[31:0] | Description Lower 4 bytes of Unique number | Attribute R | Reset By Die |
| --- | --- | --- | --- | --- |

Address: 0x1FFF 2DF8

| Bit 31:0 | Name H4BYTE[31:0] | Description High 4 bytes of Unique number | Attribute R | Reset By Die |
| --- | --- | --- | --- | --- |

SONiX TECHNOLOGY CO., LTD Page 41 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

2.7 CORE REGISTER OVERVIEW

| Register | Description (Refer to Cortex-M0 Spec) |
| --- | --- |
| R0~R12 | General-purpose registers for data operations. |
| SP (R13) | The Stack Pointer (SP). In Thread mode, the CONTROL register indicates the stack pointer to use, Main Stack Pointer (MSP) or Process Stack Pointer (PSP) On reset, the processor loads the MSP with the value from address 0x00000000. |
| LR (R14) | The Link Register (LR). It stores the return information for subroutines, function calls, and exceptions. |
| PC (R15) | The Program Counter (PC). It contains the current program address. On reset, the processor loads the PC with the value of the reset vector, at address 0x00000004. |
| PSR | The Program Status Register (PSR) combines: • Application Program Status Register (APSR) • Interrupt Program Status Register (IPSR) • Execution Program Status Register (EPSR). These registers are mutually exclusive bit fields in the 32-bit PSR. |
| PRIMASK | The PRIMASK register prevents activation of all exceptions with configurable priority. |
| CONTROL | The CONTROL register controls the stack used when the processor is in Thread mode. |

SONiX TECHNOLOGY CO., LTD Page 42 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

SYSTEM CONTROL

3.1 RESET

A system reset is generated when one of the following events occurs:

1. A low level on the RST pin (external reset).

2. Power-on reset (POR reset)

3. LVD reset

4. Watchdog Timer reset (WDT reset)

5. Software reset (SW reset)

The reset source can be identified by checking the reset flags in System Reset Status register (SYS0_RSTST). These sources act on the RST pin and it is always kept low during the delay phase. The RESET service routine vector  is fixed at address 0x00000004 in the memory map. For more details, refer to Interrupt and Exception Vectors.

Finishing any reset sequence needs some time. The system provides complete procedures to make the power on reset  successful. For different oscillator types, the reset time is different. That causes the VDD rise rate and start-up time of

timing diagram is as following.

different oscillator is not fixed. RC type oscillator’s start-up time is very short, but the crystal type is longer. Under client  terminal application, users have to take care of the power on reset time for the master terminal requirement. The reset

Power

External Reset

VDD

VSS VDD

VSS

LVD Detect Level

External Reset

Low Detect

External Reset  High Detect

Watchdog

Watchdog Reset System Status

Watchdog Normal Run

Watchdog Stop

System Normal Run System Stop

Power On Delay Time

External

Reset Delay Time

Overflow

Watchdog

Reset Delay

Time

3.1.1 POWER-ON RESET (POR)

The power on reset depends on LVD operation for most power-up situations. The power supplying to system is a rising  curve and needs some time to achieve the normal voltage. Power on reset sequence is as following:

⮚ Power-up: System detects the power voltage up and waits for power stable.

⮚ External reset (only external reset pin enable): System checks external reset pin status. If external reset pin is  not high level, the system keeps reset status and waits external reset pin released.

⮚ System initialization: All system registers is set as initial conditions and system is ready. ⮚ Oscillator warm up: Oscillator operation is successfully and supply to system clock.

⮚ Program executing: Power on sequence is finished and program executes from Boot loader.

SONiX TECHNOLOGY CO., LTD Page 43 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

3.1.2 WATCHDOG RESET (WDT RESET)

Watchdog reset is a system protection. In normal condition, system works well and clears watchdog timer by program.  Under error condition, system is in unknown situation and watchdog can’t be clear by program before watchdog timer  overflow. Watchdog timer overflow occurs and the system is reset. After watchdog reset, the system restarts and returns  normal mode. Watchdog reset sequence is as following.

● Watchdog timer status: System checks watchdog timer overflow status. If watchdog timer overflow occurs, the  system is reset.

● System initialization: All system registers is set as initial conditions and system is ready. ● Oscillator warm up: Oscillator operation is successfully and supply to system clock.

● Program executing: Power on sequence is finished and program executes from 0x0. Watchdog timer application note is as following.

● Before clearing watchdog timer, check I/O status and check RAM contents can improve system error. ● Don’t clear watchdog timer in interrupt vector and interrupt service routine. That can improve main routine fail. ● Clearing watchdog timer program is only at one part of the program. This way is the best structure to enhance the  watchdog timer function.

✴ Note: Please refer to the “WATCHDOG TIMER” about watchdog timer detail information.

3.1.3 BROWN-OUT RESET

3.1.3.1 BROWN OUT DESCRIPTION

The brown-out reset is a power dropping condition. The power drops from normal voltage to low voltage by external  or executing program error.

factors (e.g. EFT interference or external loading changed). The brown out reset would make the system not work well  VDD

System Work

VSS

V1

V2 V3

Brown-Out Reset Diagram

Well Area

System Work Error Area

The power dropping might through the voltage range that’s the system dead-band. The dead-band means the power  range can’t offer the system minimum operation power requirement. The above diagram is a typical brown out reset  diagram. There is a serious noise under the VDD, and VDD voltage drops very deep. There is a dotted line to separate  the system working area. The above area is the system work well area. The below area is the system work error area  called dead-band. V1 doesn’t touch the below area and not affect the system operation. But the V2 and V3 is under the  below area and may induce the system error occurrence. Let system under dead-band includes some conditions.

DC application:

The power source of DC application is usually using battery. When low battery condition and MCU drive any loading, the  power drops and keeps in dead-band. Under the situation, the power won’t drop deeper and not touch the system reset  voltage. That makes the system under dead-band.

SONiX TECHNOLOGY CO., LTD Page 44 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

AC application:

In AC power application, the DC power is regulated from AC power source. This kind of power usually couples with AC  noise that makes the DC power dirty. Or the external loading is very heavy, e.g. driving motor. The loading operating induces noise and overlaps with the DC power. VDD drops by the noise, and the system works under unstable power  situation.

The power on duration and power down duration are longer in AC application. The system power on sequence protects  the power on successful, but the power down situation is like DC low battery condition. When turn off the AC power, the  VDD drops slowly and through the dead-band for a while.

3.1.3.2 THE SYSTEM OPERATING VOLTAGE DECSRIPTION

To improve the brown out reset needs to know the system minimum operating voltage which is depend on the system  electrical characteristic section shows the system voltage to executing rate relationship.

executing rate and power level. Different system executing rates have different system minimum operating voltage. The  System Mini.

Vdd (V)

Normal Operating Area

Operating Voltage.

Reset Area

Dead-Band Area System Rate (Fcpu)

System Reset Voltage.

Normally the system operation voltage area is higher than the system reset voltage to VDD, and the reset voltage is  decided by LVD detect level. The system minimum operating voltage rises when the system executing rate upper even  higher than system reset voltage. The dead-band definition is the system minimum operating voltage above the system  reset voltage.

3.1.3.3 BROWN-OUT RESET IMPROVEMENT

How to improve the brown reset condition? There are some methods to improve brown out reset as following.

● LVD reset

● Watchdog reset

● Reduce the system executing rate

● External reset circuit. (Zener diode reset circuit, Voltage bias reset circuit, External reset IC)

✴ Note: The “Zener diode reset circuit”, “Voltage bias reset circuit” and “External reset IC” can completely improve the brown out reset, DC low battery and AC slow power down conditions.

SONiX TECHNOLOGY CO., LTD Page 45 Version 1.1

SN32F400 Series

LVD reset:

VDD

VSS

Power

LVD Detect Voltage

Power is below LVD Detect Voltage and System Reset.

32-Bit Cortex-M0 Micro-Controller

System Status

System Normal Run System Stop

Power On Delay Time

The LVD (low voltage detector) is built-in SONiX 32-bit MCU to be brown out reset protection. When the VDD drops and  is below LVD detect voltage, the LVD asserts an interrupt signal to the NVIC. This signal can be enabled for interrupt in  the Interrupt Enable Register in the NVIC in order to cause a CPU interrupt; if not, SW can monitor the signal by reading  a dedicated status register. An additional threshold level can be selected to cause a forced reset of the chip. The LVD  detect level is different by each MCU. The LVD voltage level is a point of voltage and not easy to cover all dead-band  range. Using LVD to improve brown out reset is dependent on application requirement and environment. If the power  variation is very deep, violent and trigger the LVD, the LVD can be the protection. If the power variation can touch the  LVD detect level and make system work error, the LVD can’t be the protection and need to other reset methods. More  detail LVD information is in the electrical characteristic section.

Watchdog reset:

The watchdog timer is a protection to make sure the system executes well. Normally the watchdog timer would be clear  at one point of program. Don’t clear the watchdog timer in several addresses. The system executes normally and the  watchdog won’t reset system. When the system is under dead-band and the execution error, the watchdog timer can’t  be clear by program. The watchdog is continuously counting until overflow occurrence. The overflow signal of watchdog  timer triggers the system to reset and return to normal mode after reset sequence. This method also can improve brown  out reset condition and make sure the system to return normal mode.

If the system reset by watchdog and the power is still in dead-band, the system reset sequence won’t be successful and  the system stays in reset status until the power return to normal range.

Reduce the system executing rate:

If the system rate is fast and the dead-band exists, to reduce the system executing rate can improve the dead-band.  The lower system rate is with lower minimum operating voltage. Select the power voltage that’s no dead-band issue  and find out the mapping system rate. Adjust the system rate to the value and the system exits the dead-band issue.  This way needs to modify whole program timing to fit the application requirement.

External reset circuit:

The external reset methods also can improve brown out reset and is the complete solution. There are three external  reset circuits to improve brown out reset including “Zener diode reset circuit”, “Voltage bias reset circuit” and “External  reset IC”. These three reset structures use external reset signal and control to make sure the MCU be reset under power  dropping and under dead-band. The external reset information is described in the next section.

3.1.4 EXTERNAL RESET

External reset function is controlled by External RESET pin control (SYS0_EXRSTCTRL) register. Default value is 1,  which means external reset function is disabled. External reset pin is Schmitt Trigger structure and low level active. The  system is running when reset pin is high level voltage input. The reset pin receives the low voltage and the system is  reset. The external reset operation actives in power on and normal running mode. During system power-up, the external  reset pin must be high level input, or the system keeps in reset status. External reset sequence is as following.

● External reset (only external reset pin enable): System checks external reset pin status. If external reset pin is  not high level, the system keeps reset status and waits external reset pin released.

● System initialization: All system registers is set as initial conditions and system is ready.  ● Oscillator warm up: Oscillator operation is successfully and supply to system clock.  ● Program executing: Power on sequence is finished and program executes from Boot loader.

SONiX TECHNOLOGY CO., LTD Page 46 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

The external reset can reset the system during power on duration, and good external reset circuit can protect the  system to avoid working at unusual power condition, e.g. brown out reset in AC power application.

3.1.4.1 SIMPLY RC RESET CIRCUIT

R1 47K ohm

C1

0.1uF

VDD RST VSS

MCU

VCC

GND

This is the basic reset circuit, and only includes R1 and C1. The RC circuit operation makes a slow rising signal into reset  pin as power up. The reset signal is slower than VDD power up timing, and system occurs a power on signal from the  timing difference.

✴ Note: The reset circuit is no any protection against unusual power or brown out reset.

3.1.4.2 DIODE & RC RESET CIRCUIT R1

DIODE

VDD

C1

0.1uF

47K ohm R2

100 ohm

RST VSS

MCU

VCC

GND

This is the better reset circuit. The R1 and C1 circuit operation is like the simply reset circuit to make a power on signal.  The reset circuit has a simply protection against unusual power. The diode offers a power positive path to conduct higher  power to VDD. It is can make reset pin voltage level to synchronize with VDD voltage. The structure can improve slight  brown out reset condition.

SONiX TECHNOLOGY CO., LTD Page 47 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

✴ Note: The R2 100 ohm resistor of “Simply reset circuit” and “Diode & RC reset circuit” is necessary to limit any current flowing into reset pin from external capacitor C in the event of reset pin breakdown due to Electrostatic Discharge (ESD) or Electrical Over-stress (EOS).

3.1.4.3 ZENER DIODE RESET CIRCUIT

R1

33K ohm

R2

10K ohm B

E C

VDD Q1

RST

MCU

Vz

R3

40K ohm

VSS

VCC

GND

The Zener diode reset circuit is a simple low voltage detector and can improve brown out reset condition completely.  Use Zener voltage to be the active level. When VDD voltage level is above “Vz + 0.7V”, the C terminal of the PNP  transistor outputs high voltage and MCU operates normally. When VDD is below “Vz + 0.7V”, the C terminal of the PNP  transistor outputs low voltage and MCU is in reset mode. Decide the reset detect voltage by Zener specification. Select  the right Zener voltage to conform the application.

3.1.4.4 VOLTAGE BIAS RESET CIRCUIT R1

47K ohm

B

R2

E

C

R3

VDD Q1

RST

MCU

10K ohm

2K ohm

VSS

VCC

GND

SONiX TECHNOLOGY CO., LTD Page 48 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

The voltage bias reset circuit is a low cost voltage detector and can improve brown out reset condition completely.  The operating voltage is not accurate as Zener diode reset circuit. Use R1, R2 bias voltage to be the active level. When  VDD voltage level is above or equal to “0.7V x (R1 + R2) / R1”, the C terminal of the PNP transistor outputs high voltage  and MCU operates normally. When VDD is below “0.7V x (R1 + R2) / R1”, the C terminal of the PNP transistor outputs  low voltage and MCU is in reset mode.

Decide the reset detect voltage by R1, R2 resistances. Select the right R1, R2 value to conform the application. In the  circuit diagram condition, the MCU’s reset pin level varies with VDD voltage variation, and the differential voltage is 0.7V. If the VDD drops and the voltage lower than reset pin detect level, the system would be reset. If want to make the reset  active earlier, set the R2 > R1 and the cap between VDD and C terminal voltage is larger than 0.7V. The external reset  circuit is with a stable current through R1 and R2. For power consumption issue application, e.g. DC power system, the  current must be considered to whole system power consumption.

✴ Note: Under unstable power condition as brown out reset, “Zener diode reset circuit” and “Voltage bias reset circuit” can protects system no any error occurrence as power dropping. When power drops below the reset detect voltage, the system reset would be triggered, and then system executes reset sequence. That makes sure the system work well under unstable power situation.

3.1.4.5 EXTERNAL RESET IC

VDD

MCU

Bypass

Capacitor 0.1uF

VDD

RST

Reset RST IC

VSS

VSS

VCC

GND

The external reset circuit also uses external reset IC to enhance MCU reset performance. This is a high cost and good  effect solution. By different application and system requirement to select suitable reset IC. The reset circuit can improve  all power variation.

3.1.5 SOFTWARE RESET

The entire MCU, including the core, can be reset by software by setting the SYSRESREQ bit in the AIRC (Application  Interrupt and Reset Control) register in Cortex-M0 spec.

The software-initiated system reset sequence is as follows:

1. A software reset is initiated by setting the SYSRESREQ bit.

2. An internal reset is asserted.

3. The internal reset is deasserted and the MCU loads from memory the initial stack pointer, the initial program counter,  and the first instruction designated by the program counter, and then begins execution.

SONiX TECHNOLOGY CO., LTD Page 49 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

3.2 SYSTEM CLOCK

Different clock sources can be used to drive the system clock (SYSCLK):

⮚ 48 MHz internal high speed RC (IHRC)

⮚ 32 KHz internal low speed RC (ILRC)

⮚ PLL clock

⮚ High speed external (EHS) crystal clock

⮚ Low speed external (ELS) 32.768 KHz crystal

Each clock source can be switched on or off independently when it is not used, to optimize power consumption.

The micro-controller is a dual clock system. There are high-speed clock and low-speed clock. The high-speed clock is  generated from the external oscillator & on-chip PLL circuit. The low-speed clock is generated from on-chip low-speed  RC oscillator circuit (ILRC 32KHz).

3.2.1 INTERNAL RC CLOCK SOURCE

3.2.1.1 Internal High-speed RC Oscillator (IHRC)

The internal high-speed oscillator is 48MHz RC type. The accuracy is ±2.5% under commercial condition. The IHRC can be switched on and off using the IHRCEN bit in Analog Block Control register (SYS0_ANBCTRL).

3.2.1.2 Internal Low-speed RC Oscillator (ILRC)

The system low clock source is the internal low-speed oscillator built in the micro-controller. The low-speed oscillator  uses RC type oscillator circuit. The frequency is affected by the voltage and temperature of the system. In common  condition, the frequency of the RC oscillator is about 32 KHz.

✴ Note: The ILRC can ONLY be switched on and off by HW.

3.2.2 EXTERNAL CLOCK SOURCE

3.2.2.1 External High-speed (EHS) Clock

decides reset time length.

External high clock includes Crystal/Ceramic modules. The startup time of Crystal is longer. The oscillator start-up time  4MHz Crystal 4MHz Ceramic

SONiX TECHNOLOGY CO., LTD Page 50 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

different.

Crystal/Ceramic devices are driven by XIN, XOUT pins. For high/normal/low frequency, the driving currents are  XIN

C

20pF

CRYSTAL

XOUT C

20pF

MCU VSS

VDD

VCC

GND

✴ Note: Connect the Crystal/Ceramic and C as near as possible to the XIN/XOUT/VSS pins of MCU.

● Structure: 4MHz~25MHz EHS external crystal/ceramic resonator

● Main Purpose: System high clock source, RTC clock source, and PLL clock source.

● Warm-up Time: 2048*FEHS

● XIN/XOUT Shared Pin Selection:

| Oscillator Mode | XIN pin | XOUT pin |
| --- | --- | --- |
| IHRC | GPIO | GPIO |
| EHS X’TAL | Crystal/Ceramic | Crystal/Ceramic |

The resonator and the load capacitors have to be placed as close as possible to the oscillator pins in order to minimize  output distortion and startup stabilization time. The loading capacitance values must be adjusted according to the  selected oscillator.

The EHS crystal is switched on and off using the EHSEN bit in Analog Block Control register (SYS0_ANBCTRL).

3.2.3 External Low-speed (ELS) Clock

The low-speed oscillator can use 32768Hz crystal oscillator circuit.

Crystal devices are driven by LXIN, LXOUT pins. The 32768 crystal and 10pF capacitor must be as near as possible to  MCU. The ELS crystal is switched on and off using the ELSEN bit in Analog Block Control register (SYS0_ANBCTRL).

SONiX TECHNOLOGY CO., LTD Page 51 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

LXIN

C

10pF

32768Hz

LXOUT C

10pF

MCU VSS

VDD

VCC

GND

✴ Note: Connect the Crystal/Ceramic and C as near as possible to the LXIN/LXOUT/VSS pins of MCU. The capacitor between LXIN/LXOUT and VSS must be 10pF.

✴ Note: When the ELS operates above 105℃ and VDD is less than 3V, a 15MΩ resistor needs to be connected in parallel to LXIN/LXOUT to ensure normal start-up.

3.2.3.1 Bypass Mode

| Clock Source | H/W Configuration | Description |
| --- | --- | --- |
| External clock source (Bypass) |  | In Bypass mode, the external clock signal (square, sinus or triangle) with ~50% duty cycle must be provided to drive the XIN/LXIN pin while the XOUT/LXOUT pin should be the inverse of the input clock signal. EHS X’tal can have a frequency of up to 25 MHz Select this mode by setting EHSEN bit in Analog Block Control register (SYS0_ANBCTRL). ELS X’TAL must have a frequency of 32.768 KHz. You select this mode by setting ELSEN bit in Analog Block Control register (SYS0_ANBCTRL). |
| External X’TAL (EHS/ELS X’TAL) |  | The 4 to 25 MHz EHS X’TAL has the advantage of producing a very accurate rate on the main clock ELS X’TAL must have a frequency of 32.768 KHz. |

SONiX TECHNOLOGY CO., LTD Page 52 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

3.2.4 PLL

SONiX 32-bit MCU uses the PLL to create the clocks for the core and peripherals. The input frequency range is 4MHz  to 25MHz. The input clock is divided down and fed to the Phase-Frequency Detector (PFD). This block compares the  phase and frequency of its inputs, and generates a control signal when phase and/ or frequency do not match. The loop  filter filters these control signals and drives the voltage controlled oscillator (VCO), which generates the main clock and  optionally two additional phases. The VCO frequency range is 96MHz to 120MHz. These clocks are divided by P by the  programmable post divider to create the output clock(s). The VCO output clock is then divided by M by the programmable  feedback divider to generate the feedback clock. The output signal of the phase-frequency detector is also monitored by  The PLL settling time is 100 μs.

the lock detector, to signal when the PLL has locked on to the input clock.

P FCLKOUT FCLKIN Fvco Pre-DIV F

PFD LPF VCO

Loop DIV

M

3.2.4.1 PLL Frequency selection

The PLL frequency equations:

FVCO = FCLKIN / F * M

FCLKOUT = FVCO / P

The PLL frequency is determined by the following parameters: ⮚ FCLKIN: Frequency from the PLLCLKSEL multiplexer.

⮚ FVCO: Frequency of the Voltage Controlled Oscillator (VCO); 96 to 120 MHz ⮚ FCLKOUT: Frequency of PLL output.

DIV

PCLK of PWM

⮚ P: System PLL post divider ratio, controlled by PSEL bits in PLL control register (SYS0_PLLCTRL). ⮚ F: System PLL front divider ratio, controlled by FSEL bits in PLL control register (SYS0_PLLCTRL). ⮚ M: System PLL feedback divider ratio, controlled by MSEL bits in PLL control register (SYS0_PLLCTRL).

To select the appropriate values for M, P, and F, it is recommended to follow these constraints: 1. 4MHz ≤ FCLKIN ≤ 25MHz

2. 4MHz ≤ (FCLKIN / F) ≤ 16MHz

3. 96MHz ≤ FVCO ≤ 120MHz

4. 1 ≤ F ≤ 3

5. 2 ≤ M ≤31

6. P = 2 or 4

7. FCLKOUT = 24MHz, 30MHz,48MHz, 50MHz, 60MHz with jitter < ±500 ps

SONiX TECHNOLOGY CO., LTD Page 53 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

3.2.4.2 PLL Recommended Frequency settings

| FCLKIN (MHz) | F[1:0] 1~3 | FCLKIN / F 4~16 MHz | M[4:0] 2~31 | FVCO(MHz)= FCLKIN / F * M 96 ~ 120 MHz | PSEL | P | FCLKOUT(MHz)= FVCO / P |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 1 | 4 | 30 | 120 | 0 | 2 | 60 |
| 4 | 1 | 4 | 30 | 120 | 1 | 4 | 30 |
| 10 | 1 | 10 | 12 | 120 | 0 | 2 | 60 |
| 10 | 1 | 10 | 10 | 100 | 0 | 2 | 50 |
| 10 | 1 | 10 | 12 | 120 | 1 | 4 | 30 |
| 12 | 1 | 12 | 8 | 96 | 0 | 2 | 48 |
| 12 | 1 | 12 | 10 | 120 | 0 | 2 | 60 |
| 16 | 1 | 16 | 6 | 96 | 0 | 2 | 48 |
| 25 | 2 | 12.5 | 8 | 100 | 0 | 2 | 50 |

3.2.5 SYSTEM CLOCK (SYSCLK) SELECTION

After a system reset, the IHRC is selected as system clock. When a clock source is used directly or through the PLL as  system clock, it is not possible to stop it.

A switch from one clock source to another occurs only if the target clock source is ready (clock stable after startup delay  or PLL locked). If a clock source which is not yet ready is selected, the switch will occur when the clock source is ready.

Ready bits in SYS0_CSST register indicate which clock(s) is (are) ready and SYSCLKST bits in SYS0_CLKCFG register indicate which clock is currently used as system clock.

3.2.6 CLOCK-OUT CAPABITITY

The MCU clock output (CLKOUT) capability allows the clock to be output onto the external CLKOUT pin. The  configuration registers of the corresponding GPIO port must be programmed in alternate function mode.

One of 6 clock signals can be selected as clock output:

1. HCLK

2. IHRC

3. ILRC

4. PLL clock output

5. ELS X’TAL

6. EHS X’TAL

The selection is controlled by the CLKOUTSEL bits in SYS1_AHBCLKEN register.

SONiX TECHNOLOGY CO., LTD Page 54 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

3.3 SYSTEM CONTROL REGISTERS 0

Base Address: 0x4006 0000

3.3.1 Analog Block Control register (SYS0_ANBCTRL)

Address Offset: 0x00

Reset value: 0x0000 0001

✴ Note: EHSEN / ELSEN / IHRCEN bit can NOT be cleared if the EHS X’tal / ELS X’tal / IHRC is selected as system clock or is selected to become the system clock.

| Bit 31:6 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 5 | EHSFREQ | Frequency range (driving ability) of EHS X’TAL 0: <=4MHz 1: >4MHz | R/W | 0 |
| 4 | EHSEN | External high-speed clock enable 0: Disable EHS X’TAL. 1: Enable EHS X’TAL. | R/W | 0 |
| 3 | Reserved |  | R | 0 |
| 2 | ELSEN | External low-speed oscillator enable 0: Disable External 32.768 KHz oscillator 1: Enable External 32.768 KHz oscillator | R/W | 0 |
| 1 | IHRCFREQ | IHRC frequency 0: IHRC 12MHz 1: IHRC 48MHz | RW | 0 |
| 0 | IHRCEN | Internal high-speed clock enable 0: Disable internal 48 MHz RC oscillator. 1: Enable internal 48 MHz RC oscillator. | R/W | 1 |

3.3.2 PLL control register (SYS0_PLLCTRL)

Address Offset: 0x04

✴ Note: PLLEN bit can NOT be cleared if the PLL is selected as system clock or is selected to become the system clock.

| Bit 31:16 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 15 | PLLEN | PLL enable 0: Disable 1: Enable | R/W | 0 |
| 14:13 | Reserved |  | R | 0 |
| 12 | PLLCLKSEL | System PLL clock source 0: IHRC 1: EHS X’TAL 4MHz~25MHz | R/W | 0 |
| 11:10 | Reserved |  | R | 0 |
| 9:8 | FSEL[1:0] | Front divider value F: 1~3 | R/W | 01b |

SONiX TECHNOLOGY CO., LTD Page 55 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

| 7:6 | Reserved |  | R | 0 |
| --- | --- | --- | --- | --- |
| 5 | PSEL | Post divider value. 0: P=2 1: P=4 | R/W | 0 |
| 4:0 | MSEL[4:0] | Loop divider value. M: 2~31 | R/W | 01000b |

3.3.2.1 RECOMMEND FREQUENCY SETTING

FVCO = FCLKIN / F * M

FCLKOUT = FVCO / P

| FCLKIN (MHz) | F[1:0] 1~3 | FCLKIN / F 4~16 MHz | M[4:0] 2~31 | FVCO(MHz)= FCLKIN / F * M 96 ~ 120 MHz | PSEL | P | FCLKOUT(MHz)= FVCO / P |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 1 | 4 | 30 | 120 | 0 | 2 | 60 |
| 4 | 1 | 4 | 30 | 120 | 1 | 4 | 30 |
| 10 | 1 | 10 | 12 | 120 | 0 | 2 | 60 |
| 10 | 1 | 10 | 10 | 100 | 0 | 2 | 50 |
| 10 | 1 | 10 | 12 | 120 | 1 | 4 | 30 |
| 12 | 1 | 12 | 8 | 96 | 0 | 2 | 48 |
| 12 | 1 | 12 | 10 | 120 | 0 | 2 | 60 |
| 16 | 1 | 16 | 6 | 96 | 0 | 2 | 48 |
| 25 | 2 | 12.5 | 8 | 100 | 0 | 2 | 50 |

3.3.3 Clock Source Status register (SYS0_CSST)

Address Offset: 0x08

| Bit 31:7 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 6 | PLLRDY | PLL clock ready flag 0: PLL unlocked 1: PLL locked | R | 0 |
| 5 | Reserved |  | R | 0 |
| 4 | EHSRDY | External high-speed clock ready flag 0: EHS oscillator not ready 1: EHS oscillator ready | R | 0 |
| 3 | Reserved |  | R | 0 |
| 2 | ELSRDY | External low-speed clock ready flag 0: ELS oscillator not ready 1: ELS oscillator ready | R | 0 |
| 1 | Reserved |  | R | 0 |
| 0 | IHRCRDY | IHRC ready flag 0: IHRC not ready 1: IHRC ready | R | 1 |

SONiX TECHNOLOGY CO., LTD Page 56 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

3.3.4 System Clock Configuration register (SYS0_CLKCFG)

Address Offset: 0x0C

| Bit 31:7 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 6:4 | SYSCLKST[2:0] | System clock switch status Set and cleared by HW to indicate which clock source is used as system clock. 000: IHRC is used as system clock 001: ILRC is used as system clock 010: EHS X’TAL is used as system clock 011: ELS X’TAL is used as system clock 100: PLL is used as system clock Other: Reserved | R | 0 |
| 3 | Reserved |  | R | 0 |
| 2:0 | SYSCLKSEL[2:0] | System clock switch Set and cleared by SW. 000: IHRC 001: ILRC 010: EHS X’TAL 011: ELS X’TAL 100: PLL output Other: Reserved | R/W | 0 |

3.3.5 AHB Clock Prescale register (SYS0_AHBCP)

Address Offset: 0x10

| Bit 31:4 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 3 | DIV1P5 | SYSCLK prescaler 0: SYSCLK = SYSCLK clock source / 1 1: SYSCLK = SYSCLK clock source / 1.5 | R/W | 0 |
| 2:0 | AHBPRE[3:0] | AHB clock source prescale value 000: SYSCLK / 1 001: SYSCLK / 2 010: SYSCLK / 4 011: SYSCLK / 8 100: SYSCLK / 16 101: SYSCLK / 32 110: SYSCLK / 64 111: SYSCLK / 128 | R/W | 0 |

3.3.6 System Reset Status register (SYS0_RSTST)

Address Offset: 0x14

This register contains the reset source except DPDWAKEUP reset, since the MODE bits in PMU_CTRL register had presented this  case.

| Bit 31:5 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 4 | PORRSTF | POR reset flag Set by HW when a POR reset occurs. 0: Read🡪No POR reset occurred Write🡪Clear this bit 1: POR reset occurred. | R/W | 1 |
| 3 | EXTRSTF | External reset flag Set by HW when a reset from the RESET pin occurs. 0: Read🡪No reset from RESET pin occurred Write🡪Clear this bit | R/W | 0 |

SONiX TECHNOLOGY CO., LTD Page 57 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

|  |  | 1: Reset from RESET pin occurred. |  |  |
| --- | --- | --- | --- | --- |
| 2 | LVDRSTF | LVD reset flag Set by HW when a LVD reset occurs. 0: Read🡪No LVD reset occurred Write🡪Clear this bit 1: LVD reset occurred. | R/W | 0 |
| 1 | WDTRSTF | WDT reset flag Set by HW when a WDT reset occurs. 0: Read🡪No watchdog reset occurred Write🡪Clear this bit 1: Watchdog reset occurred. | R/W | 0 |
| 0 | SWRSTF | Software reset flag Set by HW when a software reset occurs. 0: Read🡪No software reset occurred Write🡪Clear this bit 1: Software reset occurred. | R/W | 1 |

3.3.7 LVD Control register (SYS0_LVDCTRL)

Address Offset: 0x18

The LVD control register selects six separate threshold values for generating a LVD interrupt to the NVIC or LVD reset.

| Bit 31:16 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 15 | LVDEN | LVD enable 0: Disable 1: Enable | R/W | 0 |
| 14 | LVDRSTEN | LVD Reset enable 0: Disable 1: Enable | R/W | 0 |
| 13:7 | Reserved |  | R | 0 |
| 6:4 | LVDINTLVL[2:0] | LVD interrupt level 0: Reserved for system 1: LVD interrupt threshold is 2.10V 2: LVD interrupt threshold is 2.50V 3: LVD interrupt threshold is 2.90V 4: LVD interrupt threshold is 3.30V 5: LVD interrupt threshold is 3.70V 6: LVD interrupt threshold is 4.10V 7: LVD interrupt threshold is 4.50V | R/W | 0 |
| 3 | Reserved |  | R | 0 |
| 2:0 | LVDRSTLVL[2:0] | LVD reset level 0: Reserved for system 1: LVD interrupt threshold is 2.10V 2: LVD interrupt threshold is 2.50V 3: LVD interrupt threshold is 2.90V 4: LVD interrupt threshold is 3.30V 5: LVD interrupt threshold is 3.70V 6: LVD interrupt threshold is 4.10V 7: LVD interrupt threshold is 4.50V | R/W | 0 |

3.3.8 External RESET Pin Control register (SYS0_EXRSTCTRL) Address Offset: 0x1C

| Bit 31:1 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 0 | RESETDIS | External RESET pin disable bit. 0: Enable external RESET pin. (P3.7 acts as RESET pin) 1: Disable. (P3.7 acts as GPIO pin) | R/W | 1 |

SONiX TECHNOLOGY CO., LTD Page 58 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

3.3.9 SWD Pin Control register (SYS0_SWDCTRL)

Address Offset: 0x20

| Bit 31:1 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 0 | SWDDIS | SWD pin disable bit. 0: Enable SWD pin. (P3.5 acts as SWDIO pin, P3.6 acts as SWCLK pin) 1: Disable. (P3.5 and P3.6 act as GPIO pins) | R/W | 0 |

3.3.10 Interrupt Vector Table Mapping register (SYS0_IVTM) Address Offset: 0x24

This register decides whether the ARM interrupt vector table is mapping to Boot ROM or User ROM

| Bit 31:16 | Name IVTMKEY[15:0] | Description IVTM register key. Read as 0. Behaviour of writing to this register is ignored unless writing 0xA5A5 to IVTMKEY at the same time. | Attribute W | Reset 0 |
| --- | --- | --- | --- | --- |
| 15:2 | Reserved |  | R | 0 |
| 1:0 | IVTM[1:0] | Interrupt table mapping selection 00: Map to Boot ROM 01: Map to User ROM Other: Reserved | R/W | By BLEN in code option |

3.3.11 Anti-EFT Ability Control register (SYS0_ANTIEFT)

Address Offset: 0x30

This register decides the HW anti-EFT ability.

| Bit 31:2 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 3:0 | AEFT[2:0] | Hold clock time 0: No 1: Level 5 2: Level 6 3: Level 7 4: Level 8 (Strongest) 9: Level 1 10: Level 2 11: Level 3 12: Level 4 Other: Reserved | R/W | 000 |

SONiX TECHNOLOGY CO., LTD Page 59 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

3.3.12 IHRC Frequency Adjustment register (SYS0_IHRCADJ) Address Offset: 0x34

Total 32 counts available for IHRC frequency adjusting and each step varies between +1% from -1%.

| Bit 31:16 | Name SYSKEY[15:0] | Description System register key Read as 0. Behaviour of writing to this register is ignored unless writing 0xA5A5 to SYSKEY at the same time. | Attribute W | Reset 0 |
| --- | --- | --- | --- | --- |
| 15:9 | Reserved |  | R | 0 |
| 8:4 | ADJ[4:0] | IHRC frequency adjusting bits. | R/W | 0 |
| 3:2 | Reserved |  | R | 0 |
| 1 | DIR | IHRC frequency adjusting direction bit 0: Positive 1: Negative | R/W | 0 |
| 0 | ADJEN | IHRC frequency adjustment enable bit 0: Disable 1: Enable | R/W | 0 |

3.3.13 CT16Bn Clock Source Select register (SYS0_CT_CLKSEL) Address Offset: 0x44

| Bit 31:7 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 6:5 | CLKSEL5[1:0] | CT16B5 PCLK source 0: HCLK 1: PLL_VCO 2: LXTAL 3: ILRC | R/W | 0 |
| 4:2 | Reserved |  | R | 0 |
| 1 | CLKSEL1 | CT16B1 PCLK source 0: HCLK 1: PLL_VCO | R/W | 0 |
| 0 | CLKSEL0 | CT16B0 PCLK source 0: HCLK 1: PLL_VCO | R/W | 0 |

SONiX TECHNOLOGY CO., LTD Page 60 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

3.4 SYSTEM CONTROL REGISTERS 1

Base Address: 0x4005 E000

3.4.1 AHB Clock Enable register (SYS1_AHBCLKEN)

Address Offset: 0x00

The SYS_AHBCLKEN register enables the AHB clock to individual system and peripheral blocks.

✴ Note: 1. When the clock is disabled, the peripheral register values may not be readable by SW and the value returned is always 0x0. 2. HW will replace GPIO with CLKOUT function directly if CLKOUTSEL is Not 0. 3. User shall disable the AHB clock for individual peripheral to decrease power consumption by demand.

| Bit 31 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 30:28 | CLKOUTSEL[2:0] | Clock output source 000: Disable 001: ILRC clock 010: ELS clock 011: Reserved 100: HCLK 101: IHRC clock (48MHz) 110: EHS clock 111: PLL clock output | R/W | 0 |
| 27 | CRCCLKEN | Enable clock for CRC. 0: Disable 1: Enable | R/W | 0 |
| 26 | FLSAHEN | Enable Flash clock during sleep mode 0: Disable during sleep mode 1: Enable during sleep mode | R | 0 |
| 25 | SRAMEN | Enable SRAM clock during sleep mode 0: Disable during sleep mode 1: Enable during sleep mode |  |  |
| 24 | WDTCLKEN | Enable clock for WDT. 0: Reserved 1: Enable | R | 1 |
| 23 | RTCCLKEN | Enable clock for RTC. 0: Disable 1: Enable | R/W | 0 |
| 22 | Reserved |  | R | 0 |
| 21 | I2C0CLKEN | Enable clock for I2C0. 0: Disable 1: Enable | R/W | 0 |
| 20 | Reserved |  | R | 0 |
| 19 | ACCCLKEN | Enable AHB clock for ACC 0: Disable 1: Enable | R/W | 0 |
| 18 | FOCCLKEN | Enable AHB clock for FOC 0: Disable 1: Enable | R/W | 0 |
| 17 | UART1CLKEN | Enable clock for UART1. 0: Disable 1: Enable | R/W | 0 |
| 16 | UART0CLKEN | Enable clock for UART0. 0: Disable 1: Enable | R/W | 0 |

SONiX TECHNOLOGY CO., LTD Page 61 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

| 15 | Reserved |  | R | 0 |
| --- | --- | --- | --- | --- |
| 14 | CMPCLKEN | Enable clock for CMP. 0: Disable 1: Enable | R/W | 0 |
| 13 | Reserved |  | R | 0 |
| 12 | SPI0CLKEN | Enable clock for SPI0. 0: Disable 1: Enable | R/W | 0 |
| 11 | ADCCLKEN | Enable clock for ADC. 0: Disable 1: Enable | R/W | 0 |
| 10 | CT16B5CLKEN | Enable clock for CT16B5. 0: Disable 1: Enable | R/W | 0 |
| 9:7 | Reserved |  | R | 0 |
| 6 | CT16B1CLKEN | Enable clock for CT16B1. 0: Disable 1: Enable | R/W | 0 |
| 5 | CT16B0CLKEN | Enable clock for CT16B0. 0: Disable 1: Enable | R/W | 0 |
| 4:3 | Reserved |  | R | 0 |
| 2 | DMA0CLKEN | Enable clock for DMA0. 0: Disable 1: Enable | R/W | 0 |
| 1 | Reserved |  | R | 0 |
| 0 | OPACLKEN | Enable clock for OPA. 0: Disable 1: Enable | R/W | 0 |

3.4.2 APB Clock Prescale register 1 (SYS1_APBCP1)

Address Offset: 0x08

✴ Note: Must reset the corresponding peripheral with SYS1_PRST register after changing the prescale value.

| Bit 31 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 30:28 | CLKOUTPRE[2:0] | Clock-out source prescaler 000: Clock-out source / 1 001: Clock-out source / 2 010: Clock-out source / 4 011: Clock-out source / 8 100: Clock-out source / 16 101: Clock-out source / 32 110: Clock-out source / 64 111: Clock-out source / 128 | R/W | 0 |
| 27:23 | Reserved |  | R | 0 |
| 22:20 | WDTPRE[2:0] | WDT clock source prescaler 000: WDT_PCLK = WDT clock source / 1 001: WDT_PCLK = WDT clock source / 2 010: WDT_PCLK = WDT clock source / 4 011: WDT_PCLK = WDT clock source / 8 100: WDT_PCLK = WDT clock source / 16 101: WDT_PCLK = WDT clock source / 32 Other: Reserved | R/W | 0 |

SONiX TECHNOLOGY CO., LTD Page 62 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

| 19:11 | Reserved |  | R | 0 |
| --- | --- | --- | --- | --- |
| 10:8 | I2C0PRE[2:0] | I2C0 clock source prescaler 000: HCLK / 1 001: HCLK / 2 010: HCLK / 4 011: HCLK / 8 100: HCLK / 16 Other: Reserved | R/W | 0 |
| 7:0 | Reserved |  | R | 0 |

3.4.3 Peripheral Reset register (SYS1_PRST)

Address Offset: 0x10

All bits are cleared by HW automatically after setting as “1”.

| Bit 31:30 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 28 | OPARST | OPA reset 0: No effect 1: Reset OPA | R/W | 0 |
| 27 | DMA0RST | DMA0 reset 0: No effect 1: Reset DMA0 | R/W | 0 |
| 26 | CRCRST | CRC reset 0: No effect 1: Reset CRC | R/W | 0 |
| 25 | Reserved |  | R | 0 |
| 24 | WDTRST | WDT reset 0: No effect 1: Reset WDT | R/W | 0 |
| 23 | RTCRST | RTC reset 0: No effect 1: Reset RTC | R/W | 0 |
| 22 | Reserved |  | R | 0 |
| 21 | I2C0RST | I2C0 reset 0: No effect 1: Reset I2C0 | R/W | 0 |
| 20 | Reserved |  | R | 0 |
| 19 | ACCRST | ACC reset 0: No effect 1: Reset ACC | R/W | 0 |
| 18 | FOCRST | FOC reset 0: No effect 1: Reset FOC | R/W | 0 |
| 17 | UART1RST | UART1 reset 0: No effect 1: Reset UART1 | R/W | 0 |
| 16 | UART0RST | UART0 reset 0: No effect 1: Reset UART0 | R/W | 0 |
| 15 | Reserved |  | R | 0 |
| 14 | CMPRST | CMP reset 0: No effect 1: Reset CMP | R/W | 0 |
| 13 | Reserved |  | R | 0 |
| 12 | SPI0RST | SPI0 reset 0: No effect 1: Reset SPI0 | R/W | 0 |
| 11 | ADCRST | ADC reset 0: No effect 1: Reset ADC | R/W | 0 |

SONiX TECHNOLOGY CO., LTD Page 63 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

| 10 | CT16B5RST | CT16B5 reset 0: No effect 1: Reset CT16B5 | R/W | 0 |
| --- | --- | --- | --- | --- |
| 9:7 | Reserved |  | R | 0 |
| 6 | CT16B1RST | CT16B1 reset 0: No effect 1: Reset CT16B1 | R/W | 0 |
| 5 | CT16B0RST | CT16B0 reset 0: No effect 1: Reset CT16B0 | R/W | 0 |
| 4 | Reserved |  | R | 0 |
| 3 | GPIOP3RST | GPIO port 3 reset 0: No effect 1: Reset GPIO port 3 | R/W | 0 |
| 2 | GPIOP2RST | GPIO port 2 reset 0: No effect 1: Reset GPIO port 2 | R/W | 0 |
| 1 | GPIOP1RST | GPIO port 1 reset 0: No effect 1: Reset GPIO port 1 | R/W | 0 |
| 0 | GPIOP0RST | GPIO port 0 reset 0: No effect 1: Reset GPIO port 0 | R/W | 0 |

3.4.4 ILRC Frequency Calibration register (SYS1_ILRCFC)

Address Offset: 0x14

| Bit 31 | Name CALEN | Description Auto-calibration function enable 0: Disable 1: Enable | Attribute R/W | Reset 0 |
| --- | --- | --- | --- | --- |
| 30 | CALDONE | Calibration done 0: Not done 1: Done | R/W | 0 |
| 29:24 | Reserved |  | R | 0 |
| 23:16 | FRQCMD[7:0] | Frequency calibration command 0x4B: Execute ILRC auto-trim function 0xB4: Load ILRC trim value Other: Reserved | W | 0 |
| 15:9 | Reserved |  | R | 0 |
| 8:0 | LFRQ[8:0] | ILRC trim value | R/W | 0x16B |

SONiX TECHNOLOGY CO., LTD Page 64 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

SYSTEM OPERATION MODE

4.1 OVERVIEW

The chip builds in 3 operating modes for difference clock rate and power saving reason. These modes control oscillators, op-code  operation and analog peripheral devices’ operation.

⮚ Normal mode

⮚ Sleep mode

⮚ Deep sleep mode

4.2 NORMAL MODE

In Normal mode, the ARM Cortex-M0 core, memories, and peripherals are clocked by the system clock. The  SYS1_AHBCLKEN register controls which peripherals are running.

Selected peripherals have individual peripheral clocks with their own clock dividers in addition to the system clock. The  peripheral clocks can be disabled respectively.

The power to various analog blocks (IHRC, EHS X’TAL, ELS X’TAL, PLL, Flash, LVD, ADC) can be controlled at any  time individually through the enable bit of all blocks.

4.3 LOW-POWER MODES

There are 2 special modes of processor power reduction: Sleep mode, and Deep-sleep mode. The PMU_CTRL register  controls which mode is desired.

The CPU clock rate may also be controlled as needed by changing clock sources, re-configuring PLL values, and/or  altering the system clock divider value. This allows a trade-off of power versus processing speed based on application  requirements.

Run-time power control allows disable the clocks to individual on-chip peripherals, allowing fine tuning of power  consumption by eliminating all dynamic power use in any peripherals that are not required for the application. Selected  peripherals have their own clock divider for power control.

✴ Note: 1. The debug mode is not supported in Deep-sleep mode. 2. The pins which are not pin-out shall be set correctly to decrease power consumption in low power modes. Strongly recommended to set these pins as input pull-up.

4.3.1 SLEEP MODE

In Sleep mode, the system clock to the ARM Cortex-M0 core is stopped and execution of instructions is suspended.

Peripheral functions, if selected to be clocked in SYS1_AHBCLKEN register, continue operation during Sleep mode and  may generate interrupts to cause the processor to resume execution. Sleep mode eliminates dynamic power used by  the processor itself, memory systems and related controllers, and internal buses.

The power state of the analog blocks (IHRC, EHS X’TAL, ELS X’TAL, PLL, Flash, LVD, ADC) is determined by the enable  bit of all blocks.

The processor state and registers, peripheral registers, and internal SRAM values are maintained and the logic levels of  the pins remain static.

SONiX TECHNOLOGY CO., LTD Page 65 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

The pins of port 1 and port 2 configured as ADC input channel must be set as input mode, inactive (no pull-down/pull-up  resistor enabled, Schmitt trigger disabled, Data register keeps low) with GPIO1_MODE, GPIO1_CFG, GPIO2_MODE and GPIO2_CFG register by program to avoid current leakage.

Wake up the chip from Sleep mode by an interrupt occurs.

The RESET pin has kept functionality in Sleep mode.

The Sleep mode is entered by using the following steps:

1. Write 4 to PMU_CTRL register.

2. Execute ARM Cortex-M0 WFI instruction.

4.3.2 DEEP-SLEEP MODE

In Deep-sleep mode, the system clock to the ARM Cortex-M0 core is stopped, and execution of instructions is suspended.

The clock to the peripheral functions are stopped because the power state of oscillators are powered down, the clock  source are stopped, except RTC low speed clock source (ELS X’tal, ILRC) if used.

✴ Note: User SHALL decide to power down low speed clock source (ELS X’TAL, ILRC oscillator) or not if RTC is enabled.

The processor state and registers, peripheral registers, and internal SRAM values are maintained and the logic levels  of the pins remain static.

All GPIO pins are served as wakeup pins. The user must program the GPIO registers for each pin to set the  appropriate edge polarity for the corresponding wakeup event, only edge sensitive is supported to wakeup MCU. The  system will exit Deep-sleep mode when GPIO indicates a GPIOn interrupt to the ARM core. Furthermore, the interrupts  corresponding to each input must be enabled in the NVIC.

The RESET pin has kept functionality in Deep-sleep mode.

The Deep-sleep mode is entered by using the following steps:

1. Write 2 to PMU_CTRL register.

2. Execute ARM WFI instruction.

The advantage of the Deep-sleep mode is that can power down clock generating blocks such as oscillators and PLL,  thereby gaining far greater dynamic power savings over Sleep mode. In addition, the Flash can be powered down in  Deep-sleep mode resulting in savings in static leakage power, however at the expense of longer wake-up times for the  Flash memory.

SONiX TECHNOLOGY CO., LTD Page 66 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

4.4 WAKEUP

4.4.1 OVERVIEW

Under low power mode, program doesn’t execute. The wakeup trigger can wake the system up to normal mode. The  wakeup function builds in interrupt operation and trigger system executing interrupt service routine as system wakeup  occurrence.

✴ The wakeup trigger sources of the Sleep mode are all interrupts and the RESET pin.

✴ The wakeup trigger sources of the Deep-sleep mode are the GPIO interrupt, RTC interrupt, CT16B5 interrupt,  and the RESET pin.

4.4.2 WAKEUP TIME

When the system is in Sleep mode, the high clock is enabled or disabled by F/W. If the high clock stops and MCU is  waken up from Sleep mode, MCU waits for 2048 external high-speed oscillator clocks and 32 internal high-speed  oscillator clocks as the wakeup time to stable the oscillator circuit. After the wakeup time, the system goes into the normal  mode.

✴ Note: Wakeup from Sleep mode spends NO wakeup time if the clock doesn’t stop.

When the system is in Deep-sleep mode, the high clock will stop. When MCU is waken up from Deep-sleep mode, MCU  waits for IHRC warm up time (10us plus 32T*IHRC) if HCLK is IHRC, or EHS Xtal warm up time (3.2ms plus 2048T*EHS) if HCLK is EHS Xtal. After the wakeup time, the system goes into the normal mode.

The value of the external high clock oscillator wakeup time from Deep sleep mode is as the following.

The total Wakeup time of EHS X’tal = 3.2ms + 1/FEHS * 2048 (sec)

⮚ Example: FEHS=20MHz, the wakeup time is as the following.

The total Wakeup time = 3.2ms + 1/FEHS * 2048 = 3.3ms (FEHS = 20MHz)

The value of the IHRC wakeup time from Deep sleep mode is as the following.

The total Wakeup time of IHRC = 10us + 1/FIHRC * 32 (sec)

⮚ Example: FIHRC=12MHz, the wakeup time is as the following.

The total Wakeup time = 10us + 1/FIHRC * 32 = 12.67 us (FIHRC = 12MHz)

✴ Note: The high clock start-up time is depended on the VDD and oscillator type of high clock.

SONiX TECHNOLOGY CO., LTD Page 67 Version 1.1

4.5 STATE MACHINE OF PMU

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

Reset

Reset condition One of reset trigger sources actives Enter mode condition 1. PMU_CTRL = 4 2. WFI instruction

Sleep mode Wake-up condition Interrupt Wake-up condition GPIO0/1/2/3 Wakeup RTC CT16B5 Reset condition One of reset trigger sources

Run

mode

actives

Enter mode condition 1. PMU_CTRL = 2 2. WFI instruction

Deep-sleep mode

SONiX TECHNOLOGY CO., LTD Page 68 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

4.6 OPERATION MODE COMPARSION TABLE

| Operation Mode | Normal Mode | Low-Power Mode |  |  |
| --- | --- | --- | --- | --- |
|  |  | Sleep Mode |  | Deep-Sleep Mode |
| IHRC | By IHRCEN |  |  | Disable |
| ILRC | ON |  |  | Enable if used as clock source of RTC/CT16B5 |
| EHS X’TAL | By EHSEN |  |  | Disable |
| ELS X’TAL | By ELSEN |  |  | Enable if used as clock source of RTC/CT16B5 |
| PLL | By PLLEN |  |  | Disable |
| Cortex-M0 | Running |  | Stop | Stop |
| Flash ROM | Enable |  | By FLASHEN bit for DMA0 | Disable |
| RAM | Enable |  | By SRAMEN bit for DMA0 | Maintain |
| ADC | By ADENB |  |  | Disable |
| LVD | By LVDEN |  |  | Disable |
| RTC | By RTCEN |  |  | By RTCEN |
| CMP | By CMnEN & CMnMODE |  |  | Disable |
| OPA | By OPnEN |  |  | Disable |
| DMA | By DMAnEN |  |  | Disable |
| Peripherals | By Enable bit of each peripheral |  |  | Disable HCLK |
| IO status | - |  | Maintained | Maintained |
| Wakeup Source | N/A |  | All interrupts, RESET pin | GPIO0/1/2/3 interrupt, CT16B5 interrupt, RTC interrupt, RESET pin |

SONiX TECHNOLOGY CO., LTD Page 69 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

4.7 PMU REGISTERS

Base Address: 0x4003 2000

4.7.1 Power Control register (PMU_CTRL)

Address Offset: 0x40

The power control register selects whether one of the ARM Cortex-M0 controlled power-down modes (Sleep mode or Deep-sleep  mode) is entered and provides the flags for Sleep or Deep-sleep modes respectively.

✴ Note: The pins which are not pin-out shall be set correctly to decrease power consumption in low power modes. Strongly recommended to set these pins as input pull-up.

| Bit 31:3 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 2:0 | MODE[2:0] | Low power mode selection 010: WFI instruction will make MCU enter Deep-sleep mode. 100: WFI instruction will make MCU enter Sleep mode. Other: Normal mode | R/W | 0 |

SONiX TECHNOLOGY CO., LTD Page 70 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

GENERAL PURPOSE I/O PORT (GPIO)

5.1 OVERVIEW

Digital ports can be configured input/output by SW

⮚ Each individual port pin can serve as external interrupt input pin.

⮚ Interrupts can be configured on single falling or rising edges and on both edges.

⮚ The I/O configuration registers control the electrical characteristics of the pads.

⮚ Internal pull-up/pull-down resistor.

⮚ Most of the I/O pins are mixed with analog pins and special function pins.

5.2 GPIO MODE

All GPIO pins are inputs and floating by default. The MODE bits in the GPIOn_CFG (n=0,1,2,3) register allow the selection  of on-chip pull-up or pull-down resistors for each pin.

SONiX TECHNOLOGY CO., LTD Page 71 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

5.3 GPIO REGISTERS

Base Address: 0x4004 4000 (GPIO 0)

0x4004 6000 (GPIO 1)

0x4004 8000 (GPIO 2)

0x4004 A000 (GPIO 3)

5.3.1 GPIO Port n Data register (GPIOn_DATA) (n=0,1,2,3)

Address offset: 0x00

| Bit 31:14 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 13:0 | DATA[13:0] | Input data (read) or output data (write) for Pn.x | R/W | 0 |

5.3.2 GPIO Port n Mode register (GPIOn_MODE) (n=0,1,2,3)

Address offset: 0x04

✴ Note: HW will switch I/O Mode directly when Specific function (Peripheral, ADC) is enabled, not through GPIOn_MODE register.

| Bit 31:14 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 13:0 | MODE[13:0] | Selects pin x as input or output (x = 0 to 13) 0: Pn.x is configured as input 1: Pn.x is configured as output | R/W | 0 |

5.3.3 GPIO Port n Configuration register (GPIOn_CFG) (n=0,1,2,3) Address offset: 0x08

Reset value: 0x00AAAAAA

✴ Note: HW will switch I/O Mode directly when Specific function (Peripheral, ADC) is enabled, not through GPIOn_MODE register.

| Bit 31:28 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 27:26 | CFG13[1:0] | Configuration of Pn.13 00: Pull-up resistor enabled. 01: Pull-down resistor enabled. 10: Inactive (no pull-down/up resistor enabled, Schmitt trigger enabled). 11: Inactive (no pull-down/up resistor enabled, Schmitt trigger disabled, data register keeps low) | R/W | 10b |
| 25:24 | CFG12[1:0] | Configuration of Pn.12 00: Pull-up resistor enabled. 01: Pull-down resistor enabled. 10: Inactive (no pull-down/up resistor enabled, Schmitt trigger enabled). 11: Inactive (no pull-down/up resistor enabled, Schmitt trigger disabled, data register keeps low) | R/W | 10b |
| 23:22 | CFG11[1:0] | Configuration of Pn.11 00: Pull-up resistor enabled. 01: Pull-down resistor enabled. 10: Inactive (no pull-down/up resistor enabled, Schmitt trigger enabled). 11: Inactive (no pull-down/up resistor enabled, Schmitt trigger disabled, data register keeps low) | R/W | 10b |

SONiX TECHNOLOGY CO., LTD Page 72 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

| 21:20 | CFG10[1:0] | Configuration of Pn.10 00: Pull-up resistor enabled. 01: Pull-down resistor enabled. 10: Inactive (no pull-down/up resistor enabled, Schmitt trigger enabled). 11: Inactive (no pull-down/up resistor enabled, Schmitt trigger disabled, data register keeps low) | R/W | 10b |
| --- | --- | --- | --- | --- |
| 19:18 | CFG9[1:0] | Configuration of Pn.9 00: Pull-up resistor enabled. 01: Pull-down resistor enabled. 10: Inactive (no pull-down/up resistor enabled, Schmitt trigger enabled). 11: Inactive (no pull-down/up resistor enabled, Schmitt trigger disabled, data register keeps low) | R/W | 10b |
| 17:16 | CFG8[1:0] | Configuration of Pn.8 00: Pull-up resistor enabled. 01: Pull-down resistor enabled. 10: Inactive (no pull-down/up resistor enabled, Schmitt trigger enabled). 11: Inactive (no pull-down/up resistor enabled, Schmitt trigger disabled, data register keeps low) | R/W | 10b |
| 15:14 | CFG7[1:0] | Configuration of Pn.7 00: Pull-up resistor enabled. 01: Pull-down resistor enabled. 10: Inactive (no pull-down/up resistor enabled, Schmitt trigger enabled). 11: Inactive (no pull-down/up resistor enabled, Schmitt trigger disabled, data register keeps low) | R/W | 10b |
| 13:12 | CFG6[1:0] | Configuration of Pn.6 00: Pull-up resistor enabled. 01: Pull-down resistor enabled. 10: Inactive (no pull-down/up resistor enabled, Schmitt trigger enabled). 11: Inactive (no pull-down/up resistor enabled, Schmitt trigger disabled, data register keeps low) | R/W | 10b |
| 11:10 | CFG5[1:0] | Configuration of Pn.5 00: Pull-up resistor enabled. 01: Pull-down resistor enabled. 10: Inactive (no pull-down/up resistor enabled, Schmitt trigger enabled). 11: Inactive (no pull-down/up resistor enabled, Schmitt trigger disabled, data register keeps low) | R/W | 10b |
| 9:8 | CFG4[1:0] | Configuration of Pn.4 00: Pull-up resistor enabled. 01: Pull-down resistor enabled. 10: Inactive (no pull-down/up resistor enabled, Schmitt trigger enabled). 11: Inactive (no pull-down/up resistor enabled, Schmitt trigger disabled, data register keeps low) | R/W | 10b |
| 7:6 | CFG3[1:0] | Configuration of Pn.3 00: Pull-up resistor enabled. 01: Pull-down resistor enabled. 10: Inactive (no pull-down/up resistor enabled, Schmitt trigger enabled). 11: Inactive (no pull-down/up resistor enabled, Schmitt trigger disabled, data register keeps low) | R/W | 10b |
| 5:4 | CFG2[1:0] | Configuration of Pn.2 00: Pull-up resistor enabled. 01: Pull-down resistor enabled. 10: Inactive (no pull-down/up resistor enabled, Schmitt trigger enabled). 11: Inactive (no pull-down/up resistor enabled, Schmitt trigger disabled, data register keeps low) | R/W | 10b |
| 3:2 | CFG1[1:0] | Configuration of Pn.1 00: Pull-up resistor enabled. 01: Pull-down resistor enabled. 10: Inactive (no pull-down/up resistor enabled, Schmitt trigger enabled). 11: Inactive (no pull-down/up resistor enabled, Schmitt trigger disabled, data register keeps low) | R/W | 10b |
| 1:0 | CFG0[1:0] | Configuration of Pn.0 00: Pull-up resistor enabled. 01: Pull-down resistor enabled. 10: Inactive (no pull-down/up resistor enabled, Schmitt trigger enabled). 11: Inactive (no pull-down/up resistor enabled, Schmitt trigger disabled, data register keeps low) | R/W | 10b |

5.3.4 GPIO Port n Interrupt Sense register (GPIOn_IS) (n=0,1,2,3) Address offset: 0x0C

| Bit 31:14 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 13:0 | IS[13:0] | Selects interrupt on pin x as level or edge sensitive (x = 0 to 13).0: Interrupt on Pn.x is configured as edge sensitive. 1: Interrupt on Pn.x is configured as event sensitive. | R/W | 0 |

SONiX TECHNOLOGY CO., LTD Page 73 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

5.3.5 GPIO Port n Interrupt Both-edge Sense register (GPIOn_IBS) (n=0,1,2,3) Address offset: 0x10

| Bit 31:14 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 13:0 | IBS[13:0] | Selects interrupt on Pn.x to be triggered on both edges (x = 0 to 13).0: Interrupt on Pn.x is controlled through register GPIOn_IEV. 1: Both edges on Pn.x trigger an interrupt. | R/W | 0 |

5.3.6 GPIO Port n Interrupt Event register (GPIOn_IEV) (n=0,1,2,3) Address offset: 0x14

| Bit 31:14 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 13:0 | IEV[13:0] | Selects interrupt on pin x to be triggered rising or falling edges (x = 0 to 13).0: Depending on setting in register GPIOn_IS, Rising edges or HIGH level on Pn.x trigger an interrupt. 1: Depending on setting in register GPIOn_IS, Falling edges or LOW level on Pn.x trigger an interrupt. | R/W | 0 |

5.3.7 GPIO Port n Interrupt Enable register (GPIOn_IE) (n=0,1,2,3) Address offset: 0x18

Bits set to HIGH in the GPIOn_IE register allow the corresponding pins to trigger their individual interrupts. Clearing a bit disables  interrupt triggering on that pin.

| Bit 31:14 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 13:0 | IE[13:0] | Selects interrupt on pin x to be enabled (x = 0 to 13). 0: Disable Interrupt on Pn.x 1: Enable Interrupt on Pn.x | R/W | 0 |

5.3.8 GPIO Port n Raw Interrupt Status register (GPIOn_RIS) (n=0,1,2,3) Address offset: 0x1C

This register indicates the status for GPIO control raw interrupts. A GPIO interrupt is sent to the interrupt controller if the corresponding  bit in GPIOn_IE register is set.

| Bit 31:14 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 13:0 | IF[13:0] | GPIO raw interrupt flag (x = 0 to 13). 0: No interrupt on Pn.x 1: Interrupt requirements met on Pn.x. | R | 0 |

SONiX TECHNOLOGY CO., LTD Page 74 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

5.3.9 GPIO Port n Interrupt Clear register (GPIOn_IC) (n=0,1,2,3) Address offset: 0x20

| Bit 31:14 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 13:0 | IC[13:0] | Selects interrupt flag on pin x to be cleared (x = 0 to 13). 0: No effect 1: Clear interrupt flag on Pn.x | W | 0 |

5.3.10 GPIO Port n Bits Set Operation register (GPIOn_BSET) (n=0,1,2,3) Address offset: 0x24

In order for SW to set GPIO bits without affecting any other pins in a single write operation, the GPIO bit is set if the corresponding bit  in the GPIOn_BSET register is set.

| Bit 31:14 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 13:0 | BSET[13:0] | Bit Set enable (x = 0 to 13) 0: No effect on Pn.x 1: Set Pn.x to “1” | W | 0 |

5.3.11 GPIO Port n Bits Clear Operation register (GPIOn_BCLR) (n=0,1,2,3) Address offset: 0x28

In order for SW to clear GPIO bits without affecting any other pins in a single write operation, the GPIO bit is cleared if the corresponding  bit in this register is set.

| Bit 31:14 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 13:0 | BCLR[13:0] | Bit clear enable (x = 0 to 13) 0: No effect on Pn.x 1: Clear Pn.x. | W | 0 |

SONiX TECHNOLOGY CO., LTD Page 75 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

PERIPHERAL FUNCTION PIN

ASSIGNMENT (PFPA)

6.1 OVERVIEW

PFPA registers are used to provide flexible assignment of digital peripheral functions to desired external pins of  different packages.

6.2 FEATURES

⮚ Flexible assignment of digital peripheral functions to desired pins.

⮚ Supported functions are UART, I2C, SPI, and PWM.

6.3 PIN ASSIGNMENT LIST

| Peripheral UART0 | Pin Name URXD0 | PA0 P0.11 | PA1 P2.0 | PA2 P3.2 | PA3 P2.10 |
| --- | --- | --- | --- | --- | --- |
|  | UTXD0 | P0.10 | P2.1 | P3.1 | P2.9 |
| UART1 | URXD1 | P1.8 | P2.3 | P2.8 | P1.1 |
|  | UTXD1 | P1.9 | P2.2 | P2.7 | P1.0 |
| SPI0 | SCK0 | P0.0 | P2.5 | P1.0 | P3.0 |
|  | SEL0 | P0.1 | P2.4 | P1.8 | P1.7 |
|  | MISO0 | P0.2 | P2.6 | P1.1 | P3.1 |
|  | MOSI0 | P0.3 | P2.7 | P1.2 | P3.2 |
| I2C0 | SCL0 | P0.6 | P1.4 | P0.10 | P1.1 |
|  | SDA0 | P0.7 | P1.5 | P0.11 | P1.2 |
| CT16B0 | PWM0 | P0.2 | P3.0 | P2.6 | P0.0 |
|  | PWM0N | P0.3 | P0.1 | P2.7 | P1.1 |
|  | PWM1 | P2.9 | P3.1 | P0.1 | P2.5 |
|  | PWM1N | P0.2 | P0.0 | P2.8 | P2.4 |
|  | PWM2 | P2.7 | P3.2 | P0.0 | P2.8 |
|  | PWM2N | P2.8 | P2.6 | P2.9 | P0.1 |
|  | PWM3 | P0.1 | P0.5 | P0.3 | P3.2 |
|  | PWM3N | P0.0 | P0.1 | P0.2 | P3.1 |
|  | BRK | P0.5 | P0.10 | P1.12. | P2.10 |
| CT16B1 | PWM0 | P2.10 | P1.1 | P1.12 | P2.0 |
|  | PWM1 | P0.5 | P1.2 | P2.8 | P2.4 |
|  | PWM2 | P0.6 | P0.10 | P2.0 | P2.7 |
|  | PWM3 | P0.7 | P0.11 | P2.1 | P2.9 |
| CT16B5 | PWM0 | P2.6 | P2.4 | P1.1 | P0.3 |
|  | PWM1 | P2.7 | P2.3 | P1.2 | P0.2 |
|  | PWM2 | P2.8 | P2.2 | P1.0 | P0.1 |

SONiX TECHNOLOGY CO., LTD Page 76 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

|  | PWM3 | P2.9 | P2.1 | P1.3 | P0.0 |
| --- | --- | --- | --- | --- | --- |
| CMP | CM0O | P2.0 | P2.6 | P1.6 | P3.0 |
|  | CM1O | P2.2 | P2.7 | P1.10 | P3.1 |
|  | CM2O | P1.0 | P0.3 | P3.8 | P1.11 |
|  | CM3O | P3.0 | P2.8 | P3.12 | P1.3 |

6.4 PFPA REGISTERS

Base Address: 0x4004 2000

6.4.1 PFPA for CT16B0 register (PFPA_CT16B0)

Address offset: 0x00

| Bit 31:30 | Name BRK | Description CT16B0_BRK assigned pin 0: P0.5 1: P0.10 2: P1.12 3: P2.10 | Attribute RW | Reset 0 |
| --- | --- | --- | --- | --- |
| 29:16 | Reserved |  | R | 0 |
| 15:14 | PWM3N | CT16B0_PWM3N assigned pin 0: P0.0 1: P0.1 2: P0.2 3: P3.1 | RW | 0 |
| 13:12 | PWM3 | CT16B0_PWM3 assigned pin 0: P0.1 1: P0.5 2: P0.3 3: P3.2 | RW | 0 |
| 11:10 | PWM2N | CT16B0_PWM2N assigned pin 0: P2.8 1: P2.6 2: P2.9 3: P0.1 | RW | 0 |
| 9:8 | PWM2 | CT16B0_PWM2 assigned pin 0: P2.7 1: P3.2 2: P0.0 3: P2.8 | RW | 0 |
| 7:6 | PWM1N | CT16B0_PWM1N assigned pin 0: P0.2 1: P0.0 2: P2.8 3: P2.4 | RW | 0 |
| 5:4 | PWM1 | CT16B0_PWM1 assigned pin 0: P2.9 1: P3.1 2: P0.1 3: P2.5 | RW | 0 |
| 3:2 | PWM0N | CT16B0_PWM0N assigned pin 0: P0.3 1: P0.1 2: P2.7 3: P1.1 | RW | 0 |
| 1:0 | PWM0 | CT16B0_PWM0 assigned pin 0: P0.2 1: P3.0 2: P2.6 3: P0.0 | RW | 0 |

SONiX TECHNOLOGY CO., LTD Page 77 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

6.4.2 PFPA for CT16B1 register (PFPA_CT16B1)

Address offset: 0x04

| Bit 31:8 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 7:6 | PWM3 | CT16B1_PWM3 assigned pin 0: P0.7 1: P0.11 2: P2.1 3: P2.9 | R/W | 0 |
| 5:4 | PWM2 | CT16B1_PWM2 assigned pin 0: P0.6 1: P0.10 2: P2.0 3: P2.7 | R/W | 0 |
| 3:2 | PWM1 | CT16B1_PWM1 assigned pin 0: P0.5 1: P1.2 2: P2.8 3: P2.4 | R/W | 0 |
| 1:0 | PWM0 | CT16B1_PWM0 assigned pin 0: P2.10 1: P1.1 2: P1.12 3: P2.0 | R/W | 0 |

6.4.3 PFPA for CT16B5 register (PFPA_CT16B5)

Address offset: 0x14

| Bit 31:8 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 7:6 | PWM3 | CT16B5_PWM3 assigned pin 0: P2.9 1: P2.1 2: P1.3 3: P0.0 | R/W | 0 |
| 5:4 | PWM2 | CT16B5_PWM2 assigned pin 0: P2.8 1: P2.2 2: P1.0 3: P0.1 | R/W | 0 |
| 3:2 | PWM1 | CT16B5_PWM1 assigned pin 0: P2.7 1: P2.3 2: P1.2 3: P0.2 | R/W | 0 |
| 1:0 | PWM0 | CT16B5_PWM0 assigned pin 0: P2.6 1: P2.4 2: P1.1 3: P0.3 | R/W | 0 |

SONiX TECHNOLOGY CO., LTD Page 78 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

6.4.4 PFPA for UART0 register (PFPA_UART0)

Address offset: 0x20

| Bit 31:4 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 3:2 | URXD0 | URXD0 assigned pin 0: P0.11 1: P2.0 2: P3.2 3: P2.10 | R/W | 0 |
| 1:0 | UTXD0 | UTXD0 assigned pin 0: P0.10 1: P2.1 2: P3.1 3: P2.9 | R/W | 0 |

6.4.5 PFPA for UART1 register (PFPA_UART1)

Address offset: 0x24

| Bit 31:4 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 3:2 | URXD1 | URXD1 assigned pin 0: P1.8 1: P2.3 2: P2.8 3: P1.1 | R/W | 0 |
| 1:0 | UTXD1 | UTXD1 assigned pin 0: P1.9 1: P2.2 2: P2.7 3: P1.0 | R/W | 0 |

6.4.6 PFPA for I2C0 register (PFPA_I2C0)

Address offset: 0x40

| Bit 31:4 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 3:2 | SCL0 | SCL0 assigned pin 0: P0.6 1: P1.4 2: P0.10 3: P1.1 | R/W | 0 |
| 1:0 | SDA0 | SDA0 assigned pin 0: P0.7 1: P1.5 2: P0.11 3: P1.2 | R/W | 0 |

SONiX TECHNOLOGY CO., LTD Page 79 Version 1.1

SN32F400 Series 32-Bit Cortex-M0 Micro-Controller

6.4.7 PFPA for SPI0 register (PFPA_SPI0)

Address offset: 0x50

| Bit 31:8 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 7:6 | SEL0 | SEL0 assigned pin 0: P0.1 1: P2.4 2: P1.8 3: P1.7 | R/W | 0 |
| 5:4 | SCK0 | SCK0 assigned pin 0: P0.0 1: P2.5 2: P1.0 3: P3.0 | R/W | 0 |
| 3:2 | MOSI0 | MOSI0 assigned pin 0: P0.3 1: P2.7 2: P1.2 3: P3.2 | R/W | 0 |
| 1:0 | MISO0 | MISO0 assigned pin 0: P0.2 1: P2.6 2: P1.1 3: P3.1 | R/W | 0 |

6.4.8 PFPA for CMP register (PFPA_CMP)

Address offset: 0x90

| Bit 31:8 | Name Reserved | Description | Attribute R | Reset 0 |
| --- | --- | --- | --- | --- |
| 7:6 | CM30 | CM3O assigned pin 0: P3.0 1: P2.8 2: P3.12 3: P1.3 | R/W | 0 |
| 5:4 | CM20 | CM2O assigned pin 0: P1.0 1: P0.3 2: P3.8 3: P1.11 | R/W | 0 |
| 3:2 | CM1I0 | CM1O assigned pin 0: P2.2 1: P2.7 2: P1.10 3: P3.1 | R/W | 0 |
| 1:0 | CM00 | CM0O assigned pin 0: P2.0 1: P2.6 2: P1.6 3: P3.0 | R/W | 0 |

SONiX TECHNOLOGY CO., LTD Page 80 Version 1.1